MARMITES × CdL

Merge implementation cookbook

Bringing the CdL modelling approach (SFR / LAK / CRR / MVR, config-driven build, PEST++-IES calibration) into MARMITES–MODFLOW 6, with a three-source total ET and a soil-moisture-aware calibration.

| Prepared for | Alain Pascal Francés — LNEG, Portugal |
| --- | --- |
| Date | 2026-09-10 |
| Source A (model approach) | MF6models — CdL/code/cdl_gwf_model_fable_v2.py (v2 “fable”), postprocess_cdl.py, the PEST chain. Local E:\00code\MF6models; remote github.com/AlainPascalFrances/MF6models |
| Source B (host code base) | MARMITES, branch MM-MF6. Local E:\00code\MARMITES; remote github.com/AlainPascalFrances/MARMITES |
| Baseline state | Updated 2026-09-10. WP0, WP1, WP1b (stage 1) and WP1c are DONE and committed on branch MM-MF6_SFR_LAK_CRR; pytest code/tests -q = 353 passed / 6 skipped. A coupled La Mata run completes on a Voronoi mesh, mass-conserving, with SFR routing over the shared-face topology. Voronoi is NOT yet the default. WP2 onwards not started. Chapter 11 says what to verify and how. |
| Companion documents | doc/MM-MF6/MARMITES_NEXT_STEPS.md (canonical running state), doc/MM-MF6/MARMITES_SFR_LAK_CRR_analysis.md §7–8 (design record) |

How to read this: this is the plan; chapter 11, at the end, is the status report -- what has actually been built, and the command that checks each claim. the work is cut into twelve work packages -- the ten numbered WP0 -> WP9, plus WP1b (the Streamlit interface) and WP1c (one grid path, Voronoi by default). Each one states its objective, the files it touches, a numbered step list, the configuration keys it introduces, and the acceptance criteria that must be green before the next package starts. Sections 6–8 are the reference material the packages point at: the input-data inventory, the configuration reference and the PEST-IES design.

# Contents

Right-click > Update Field to build the table of contents.

# 1  Purpose, scope and working method

## 1.1  What this cookbook delivers

Four things were asked for, and they are interleaved rather than sequential — so the plan orders them by dependency, not by the order they were requested:

SFR, LAK and CRR in MARMITES, plus an ET calibration with PEST++-IES whose observations are heads (MF), soil moisture at several depths (MM), streamflow (SFR) and total ET (MM+MF). → WP3, WP4, WP5, WP7.

Total ET recomposed from three sources: ETsoil from MM, ETuzf from MF (the deep unsaturated zone), ETg from MM — which means MODFLOW's own groundwater ET must stay switched off. → WP2.

A configuration file replacing the command-line tags, in the style of the CdL script, plus an inventory of the input data MM and the SFR/LAK approach need, split between the repository and an outside-the-repo data root. → WP0, WP1, §6, §7.

A Streamlit web interface to browse the input data — the shapefiles included — to launch the model locally or on the server, and to present the output. → WP1b.

Two further packages were added because leaving them out would force rework: WP6 (post-processing and the observation exports, without which WP7 has nothing to calibrate against) and WP8 (tests, documentation and the MM-MF6 → master merge hygiene).

Two more follow from the decisions of 2026-09-09 (§1.4 and §3.3): WP1c — one grid path, Voronoi by default, so La Mata and CdL share a single discretisation and the surface packages are built once — and WP9, the port to CdL. Eleven packages in all.

## 1.2  Dependency order — why WP0 and WP1 come first

WP2 to WP5 each introduce a handful of new settings and a handful of new input files. Adding them to the current argparse flag list and to an ad-hoc dataset layout, and only then refactoring to a configuration file, means writing the same settings twice and re-testing them twice. Doing the configuration and the data layout first costs about two days and is repaid by every package after it.

WP1b (the Streamlit interface) sits on both of them — it edits the configuration file WP0 defines and browses the data WP1 sorts, so it cannot sensibly be built before either. It is then delivered in three stages rather than in one, because its output side has to follow the figures WP3–WP7 produce.

WP1c comes before WP2–WP5, not after. SFR routing, LAK host cells and the CRR neighbour graph are all grid-topology dependent: build them on the raster and they are built twice.

WP0 config file    ─┬─► WP1b Streamlit UI (stage 1)
WP1 data inventory  ├─► WP1c ONE GRID PATH — Voronoi by default
                    │        │
                    │        ├─► WP2  ET (ETsoil + ETuzf + ETg)
                    │        ├─► WP3  SFR coupled + validated
                    │        │        └─► WP4  LAK coupled + validated
                    │        │                 └─► WP5  CRR (python cascade)
                    └────────┴───────────────────► WP6  post-proc + obs exports
                                                    │      └─► WP1b stage 2
                                                    └─► WP7  PEST-IES
                                                           ├─► WP1b stage 3
                                                           └─► WP9  port to CdL

                    WP8  tests / docs / merge   (runs alongside)

## 1.3  Working method (unchanged from the conversion phase)

Implement, then hand off. The transient and multi-year runs are launched by Alain in Spyder. Every hand-off must name the files to reload in Spyder before the run, and the exact configuration to set.

One real short run per step. Every package is validated against a real run (run.nsp = 365, one hydrological year) and the figures are looked at before the step is declared done. This is the discipline that ended the last restart; keep it.

Modify the native functions, never reimplement them. That rule produced the complete figure suite; it applies equally to MARMITESplot_v3.py and to the CdL post-processing being ported in WP6.

MARMITES_NEXT_STEPS.md stays the canonical running state. This cookbook is the plan; that file records what actually happened. Update its “CURRENT STATE” block at the end of every work package.

## 1.4  Study area: develop on La Mata, deliver on CdL

Decided 2026-09-09. The run-time difference — about 10 minutes at La Mata against about 30 at CdL — is the least important factor. The decisive one is data.

The blocker: CdL's forcing is monthly (p_month_198101_202605.csv, et0_month_198101_202605.csv). MARMITES is a daily model: MMsurf needs hourly meteorology — rainfall, wind speed, air humidity, temperature, incoming solar radiation — to produce the daily P / Pe / RF / RFe / TF / PT / PE / E0 / LAI series MMsoil consumes. MM cannot run on CdL today at all. It is blocked on data, not merely slower.

Three further gaps, none of them small: CdL has no soil or vegetation zoning in the MARMITES sense (the raw material exists — the ks / ths / wp / fc rasters and COS-2025 — but must be converted); MARMITES builds its grid from ASCII rasters whereas CdL's comes from voronoi_grid.pkl + voronoi_layers.npz (WP1c removes this one); and CdL's observations are not in MARMITES formats.

The calibration argument also runs the other way. La Mata being uncalibrated is not a drawback here — calibration is the deliverable — and La Mata carries the better observation set for the ET and soil-moisture calibration this project is about: real head records at 11 points, real soil-moisture probes at several depths, catchment runoff, and an eddy-covariance tower. CdL's counterparts are synthetic piezometers and MODIS AET. (Worth confirming on the CdL side: per the project record the last IES overfitted to a checkerboard K field and the DRN-datum change invalidated that parameter set, so CdL has calibration machinery rather than a current trusted calibration.)

So: WP0–WP7 on La Mata, including a real calibration; CdL enters through WP9. The configuration file and the example/<case>/ layout make the port data and a grid reader, not a rewrite. One rule protects that: nothing case-specific in code/ — if it names La Mata, it belongs in a configuration file or in example/LaMata/.

# 2  Where the two code bases stand today

## 2.1  Side-by-side

| Aspect | MARMITES (branch MM-MF6) | CdL (MF6models) |
| --- | --- | --- |
| Purpose | Physically-based soil-water balance (MMsurf + MMsoil) coupled to MF6 through the BMI/API; La Mata catchment, daily, 1949 stress periods | MF6 groundwater-flow model of the Casa de Lobos catchment; monthly 1981–2026 |
| Grid | Structured DIS (and DISV-from-DIS) inherited from the NWT model; 50 m cells. An unstructured path already exists but is unwired — VertexGeometry, RefinedModel, resample_inputs — see WP1c, which makes Voronoi the default | Voronoi/DISV via Triangle; ncpl 6609 (design 2) or 3295 (design 1) |
| Layers | --nlay 2 reads _2s1L.ini directly (6-layer set still available) | nlay = 3 — one numerical layer per geologic unit (U1/U2/U3), Daoud-style |
| Soil / unsaturated | MMsoil does the soil column in Python; UZF6 is only the percolation zone below the soil, with simulate_et=False | UZF is the soil zone — per-cell soil rasters, simulate_et + linear_gwet on |
| Seepage face | DRN-SEEP (seep='drn', cond 10000) — validated; exfiltration returned to the MM soil column | DRN-SEEP with cubic smoothing over DDRN; routed on to SFR by MVR |
| Streams | marmites_sfr.py — priority-flood routing from inputPONDw.asc; builds and reloads, never run coupled | SFR built directly on the DISV grid; DEM routing, Manning rectangular, drainage-scaled width, downstream-monotonic bed (SFRmaker rule) |
| Ponds | marmites_lak.py — 12 EMBEDDEDV lakes from lm_ponds.shp; builds, never run coupled | 19 charcas as EMBEDDEDV LAK, centroid-seeded cells, wedge stage/vol table, Manning spill outlet |
| CRR | Not started. Design decided (analysis doc R2): must run in Python, receiver = MM soil column | Implemented as static MVR movers, MFD weights α=β·S/ΣS, receivers LAK > SFR > UZF |
| ET | ETsoil + ETg both in MM; ETg applied to MF6 as a WEL sink; no UZF ET at all | All ET in UZF (unsaturated + groundwater), extinction depth = COS rooting depth |
| Coupling | BMI/API through libmf6, lagged or iterative, per stress period | None — a standalone MF6 run |
| Configuration | ~40 argparse flags on code/tests/run_lamata_mf6.py; a legacy positional .ini; a TOML schema exists (code/marmites_config.py) but only covers the MM run-control block | config.py (three machine paths) + a documented module-level settings block at the top of the script |
| Calibration | None. Drawdown deficit flagged as a calibration matter, deferred | pyEMU + pestpp-ies: pilot points, geostatistical prior, Tikhonov smoothness, heads / MODIS AET / SFR streamflow |
| Post-processing | Native MARMITESplot_v3 suite, complete: Sankeys, per-point series, calibration criteria, MM and GW maps, input parameter maps, general map | postprocess_cdl.py: water balance in mm/yr, streamflow hydrographs, head and AET fits, lake stage/volume, MVR accounting, per-pond panels |
| Repo rule | Repo = code + docs + strictly the files MM/MF read; all output to $MARMITES_WS_ROOT outside the repo | Repo carries input_data/ and a ready-to-run input_model/; large rasters documented, not committed |

## 2.2  What already exists on the MARMITES side (do not rebuild it)

code/ppMF6/marmites_sfr.py — priority-flood stream network, downstream-monotonic bed elevations, EVAPORATION = 0, outlet reaches replacing the six outlet DRN cells.

code/ppMF6/marmites_lak.py — EMBEDDEDV lakes, wedge stage–volume–area table, host-cell assignment by centroid, on-channel / inlet / outlet reach attributes.

code/ppMF6/marmites_mf6.py — MVR wiring for the stream running through the on-channel ponds; _add_sfr_package, _add_lak_package, _add_mvr_package.

code/marmites_coupler.py — the SFR INFLOW pointer is already bound and the MM runoff of channel cells is already injected as reach inflow, through the post-prepare_solve callback.

code/marmites_config.py — a validated TOML schema with an MMConfig dataclass and a legacy-ini converter. WP0 extends this rather than inventing a new mechanism.

## 2.3  What is genuinely missing

A coupled run of SFR and LAK — the packages have never been exercised through the API.

Any ET in the deep unsaturated zone (UZF), and therefore any three-source ET total.

The CRR cascade, and the descending-topographic cell ordering it needs.

A LAK↔MM exchange: lake inflow written from MM, lake stage read back so MM's open-water evaporation uses the real wetted area.

Streamflow, lake-stage and total-ET observation exports — the inputs a PEST run consumes.

Everything on the calibration side.

# 3  Target architecture

## 3.1  The merged picture

MARMITES (python, per stress period)                     MODFLOW 6 (libmf6, BMI)
  ────────────────────────────────────                     ─────────────────────────
  MMsurf   P, PE, PT, LAI, interception  ─┐
                                          │
  MMsoil   per soil layer:                │   perc (m/d)        ┌──►  UZF   SINF
            Esoil, Tsoil, Ssoil, Rsoil    ├───────────────────► │      (deep unsat zone only)
            Ssurf, I, Ro, Eow             │   PETres (m/d)      └──►  UZF   PET   ── ETuzf
            ETg = Eg + Tg                 │   ETg (m3/d)        ────►  WEL   Q
                                          │
  CRR      MFD cascade over the DEM       │   Ro_reach (m3/d)   ────►  SFR   INFLOW
            β·S_ij/ΣS_ij, sinks evaporate ├───────────────────►
            receivers: LAK > SFR > soil   │   Ro_lake (m3/d)    ────►  LAK   RUNOFF
                                          │
                            ◄─────────────┘   heads, exfiltration (DRN-SEEP),
                                              rejected infiltration, LAK stage

  TOTAL ET  =  Ei (MM)  +  Eow (MM)  +  ETsoil (MM)  +  ETuzf (MF/UZF)  +  ETg (MM, via WEL)
                                                         ▲
                                       UZF groundwater ET is OFF — no LINEAR_GWET / SQUARE_GWET

## 3.2  What crosses the API each stress period (extended)

| Direction | Quantity | Target / source | Status |
| --- | --- | --- | --- |
| MM → MF6 | percolation (m/d) | UZF SINF (never FINF) | exists |
| MM → MF6 | ETg (m³/d) | WEL Q, AUTO_FLOW_REDUCE | exists |
| MM → MF6 | runoff of channel cells (m³/d) | SFR INFLOW | exists, never run coupled |
| MM → MF6 | residual PET after MM's own ET (m/d) | UZF PET | new — WP2 |
| MM → MF6 | runoff captured by a pond (m³/d) | LAK RUNOFF | new — WP4 |
| MF6 → MM | heads (m) | X, per surface cell | exists |
| MF6 → MM | exfiltration (m³/d) | DRN-SEEP flows (or UZF GWD) | exists |
| MF6 → MM | rejected infiltration (m³/d) | UZF REJ-INF | exists |
| MF6 → MM | ETuzf actually taken (m³/d) | UZF ET budget term | new — WP2 |
| MF6 → MM | lake stage (m) → wetted area for E_ow | LAK STAGE | new — WP4 |

Timing rule (do not relearn it): every API-written input — SINF, Q, INFLOW, and now PET and LAK RUNOFF — must be written after prepare_solve, through the existing write_cb callback. UZF re-derives its arrays from the period data during both prepare_time_step and prepare_solve, so anything written earlier is silently overwritten. This is the bug that made UZF apply a constant 977 m³/d for the whole project.

## 3.3  Three decisions taken here, with their reasons

### D1 — CRR runs in Python, not as MVR movers

The CdL model routes the cascade with static MVR movers whose reinfiltration receiver is a UZF cell. In CdL that is correct, because UZF is the soil zone. In MARMITES, UZF is only the percolation zone below the soil column, so an MVR reinfiltration would inject water beneath Ssoil and bypass Esoil, Tsoil and the Eq. 1b infiltration limit — defeating the ET partitioning that is the point of the model. The cascade therefore runs in the coupler, reusing Daoud's Eq. 23 weights and the LAK > SFR > soil receiver priority; only the reinfiltration target differs. (Analysis doc, revision R2.)

### D2 — LAK connection type is EMBEDDEDV, definitively

The ponds are sub-grid (341–2036 m² against a 2500 m² cell; two contain no cell centre), so there is nothing to excavate. A VERTICAL connection puts the lakebed at the cell top, which would leave a dug-in pond permanently dry. This matches the locked CdL decision; tune inside the design (bed leakance, table, surfdep, solver settings), never the connection type.

### D3 — geospatial libraries stay off the model path

Reading hydrography.shp and lm_ponds.shp during the build would put geopandas (or pyshp, which is not installed) into the model path, where today only one optional figure needs it. Instead, a one-off converter (code/tools/gis_to_dataset.py, WP1) turns the shapefiles and rasters into small plain-text tables that live in the repository dataset and that MM/MF read directly. This answers the open dependency question in the handoff document and satisfies the repository rule at the same time.

### D4 — two infiltration returns, in opposite directions; do not merge them

Two distinct fluxes are easy to conflate, and conflating them would quietly break the soil budget. Both exist, and they chain:

|  | Rejected infiltration returning to the soil | CRR cascade reinfiltration |
| --- | --- | --- |
| What it is | MF6/UZF refuses infiltration, or groundwater exfiltrates; the water is given back to the same cell's soil column | Surface runoff Ro routed from an upslope cell to a downslope one |
| Enters at | the BOTTOM soil layer, propagating upward | the TOP soil layer |
| Governed by | saturation from below: the column fills, emerges as Exf_g1 and becomes Dunnian runoff | the infiltration law, Eq. 1b: I = min[Ssurf , D_soil,1 (φ1 − θ1)] |
| Status | already coded in MMsoil — the documented MARMITES behaviour (analysis doc §5). WP5 must not disturb it | WP5, new; D1 above is about this leg only |

The chain runs: MF6 rejects infiltration → it returns bottom-up into that cell's column → it emerges as Ro → CRR routes that Ro downslope → it reinfiltrates top-down into the receiving column, or reaches a pond or a reach. Where D1 and this agree, and it is the point that matters: the receiver is always the MARMITES soil column, never a UZF cell.

### D5 — one grid path, Voronoi by default

La Mata moves off its 50 m structured raster onto a Voronoi/DISV grid built the way CdL builds its own, so both case studies share one discretisation and the surface packages are written once. This is WP1c, placed before WP2 because SFR routing, LAK host cells and the CRR neighbour graph all depend on grid topology. It also removes D2's sub-grid problem: with pond-scale seeded cells a pond cell really is a pond, so WP4's open-water bypass becomes exact rather than approximate. The structured producer is kept permanently, as the regression anchor and for the MODFLOW-NWT comparison.

# 4  Standing rules and known traps

These are the things that have already cost time. Every step below assumes them.

| # | Rule / trap | Why it matters |
| --- | --- | --- |
| 1 | The repository holds code, docs and strictly the files MM and MF read directly. All run output goes to $MARMITES_WS_ROOT. | No GIS, no output, however small. .gitignore enforces it. |
| 2 | Never git add -A while on master. | master carries only upstream's minimal .gitignore. |
| 3 | UZF infiltration is SINF, not FINF. | FINF resolves to a valid but non-operative pointer. |
| 4 | Write every API input after prepare_solve. | See §3.2. Applies to the two new writes as well. |
| 5 | get_input_var_names() is not authoritative. | Advanced-package variables (UZF SINF) are reachable but unlisted. Validate a bound pointer by size. |
| 6 | Keep the infiltration-fidelity guard in check_solution. | It is what caught trap 3. WP2 adds a sibling guard for groundwater ET. |
| 7 | A steady-state period ignores the initial-head file. | Seed it with mean recharge/ETg (--steady-means / spinup.steady_means), not carried heads. |
| 8 | max_outer must equal the IMS OUTER_MAXIMUM. | Otherwise ATS never sees a failed step and cannot retry with a smaller dt. |
| 9 | Confirm .hds / .lst / .uzf.cbc and _coupled_*.h5 come from the same run. | A one-year cbc mixed with a full-run h5 has already wasted a diagnosis. |
| 10 | flopy: an EXTERNAL lake outlet is lakeout = -1, not 0. | The CdL transient failed on exactly this. |
| 11 | flopy get_structured_faceflows(ia=, ja=) is broken upstream (3.10 and 3.11). | _ja_down_index in marmites_postprocess.py is the replacement. Remember it if SFR/LAK post-processing needs face flows. |
| 12 | inputPONDw.asc / inputPONDhmax.asc hold the stream network, not ponds. | A legacy misnomer. WP1 renames them; the IN_007 / IN_008 maps built from them retire with SFR. |
| 13 | A configuration silently overridden by a cache is a lost run. | The CdL grid-design cache did this. WP0 stamps the resolved configuration into the run folder and fails fast on a mismatch. |
| 14 | Remind Alain to reload the edited .py files in Spyder before flipping any setting. | Every hand-off, without exception. |
| 15 | The two drain families have different datums. OUTLET drains sit at the bottom of the layer (botm[lay, cell], the CdL rule). DRN-SEEP sits at the LAND SURFACE and must stay there. | DRN-SEEP represents groundwater emerging where the water table reaches the ground; move its datum to the layer bottom and the seepage face stops working — silently, since the package still builds and the run still converges. Confirmed 2026-09-09. |

# 5  The work packages

## WP0 — A configuration file replaces the command-line tags   [DONE]

Objective. One human-readable file per model configuration, in the CdL spirit: machine paths in one place, model and run settings in a documented, validated block. The ~40 argparse flags become configuration keys; the CLI keeps only a --config argument and an explicit override switch.

### Design

Two layers, mirroring CdL but reusing what MARMITES already has:

| Layer | File | Holds | CdL equivalent |
| --- | --- | --- | --- |
| Machine paths | code/mm_paths.py | REPO, DATA_ROOT, WS_ROOT, LIBMF6, MF6_EXE, PESTPP_IES, PYTHON_EXE — each overridable by an MM_* environment variable | CdL/code/config.py |
| Model + run settings | code/configs/lamata.toml (and siblings per configuration) | everything that is a flag today, grouped in sections, plus the new SFR/LAK/CRR/ET/PEST keys | the module-level settings block at the top of cdl_gwf_model_fable_v2.py |

TOML rather than a Python module, because code/marmites_config.py already parses TOML into a validated MMConfig dataclass with a legacy-ini converter and a test file (code/tests/test_config.py). Extending that schema costs a fraction of writing a second mechanism, and it keeps the configuration data — so it can be copied into a run folder, diffed between runs, and templated by pyEMU in WP7. Machine paths stay in Python so environment variables and Path arithmetic work exactly as in CdL.

### Steps

| # | Action | Files | Done when |
| --- | --- | --- | --- |
| 0.1 | Extend the MMConfig dataclass with the new sections: [paths] [run] [grid] [layers] [uzf] [seep] [et] [sfr] [lak] [crr] [spinup] [postproc] [pest] [meta]. Give every key a default equal to today's flag default, so an empty file reproduces today's behaviour. | code/marmites_config.py | load_mm_config() round-trips a full file; unknown keys raise |
| 0.2 | Write code/mm_paths.py with the seven roots and their MM_* environment-variable overrides, plus a report_paths() that prints what resolved. | new file | importing it on a clean machine prints every root and flags the missing ones |
| 0.3 | Author code/configs/lamata.toml — the full annotated reference configuration (§7), reproducing the canonical spun-up run exactly. | new file | the run below is byte-identical to the flag-driven one |
| 0.4 | Rewrite the CLI: run_lamata_mf6.py --config code/configs/lamata.toml [--set section.key=value ...] [--run-tag TAG]. Keep --set for one-off overrides; delete the rest of the flags. Every override is echoed at startup. | code/tests/run_lamata_mf6.py | --help fits on a screen; no behaviour depends on an unprinted default |
| 0.5 | Provenance: copy the resolved configuration (after --set) into the run folder as _input/resolved_config.toml, and write its hash into the coupled HDF5 attributes. | run_lamata_mf6.py, marmites_coupler.py | every run folder says exactly what produced it |
| 0.6 | Fail-fast guards, in the spirit of the CdL design-tag sidecar: a cached grid / layer set / spin-up state carries the configuration hash it was built from; a mismatch stops with a message naming the offending key. No silent override, ever. | marmites_mf6.py, run_lamata_mf6.py | changing layers.nlay against a stale cache raises |
| 0.7 | Migration helper code/tools/flags_to_toml.py: takes an old command line, prints the equivalent TOML. | new file | the two canonical commands in MARMITES_NEXT_STEPS.md §0 convert cleanly |
| 0.8 | Extend code/tests/test_config.py: defaults, round-trip, unknown-key rejection, --set override, guard trip. | code/tests/test_config.py | suite green |

### Acceptance

python code/tests/run_lamata_mf6.py --config code/configs/lamata.toml reproduces the current canonical run (same MF6 input files, same water balance).

No model behaviour is reachable except through the configuration file or an echoed --set.

Full flag → key map published (Appendix A) and the old commands in the handoff document rewritten.

## WP1 — Input-data inventory and the DATA_ROOT split   [DONE]

Objective. Name every input MM and the SFR/LAK/CRR approach need; put the raw geospatial sources in a data root outside the repository; and generate, once, the small plain-text intermediates that MM and MF actually read — which are the only new files allowed into the repository.

The full inventory is §6. The mechanism is a single converter run by hand, never by the model:

DATA_ROOT (outside the repo)                     REPO  example/LaMata/  (read directly by MM/MF)
  ────────────────────────────                     ──────────────────────────────────────────────
  GIS/hydrography.shp        ─┐
  GIS/lm_ponds.shp            │   tools/gis_to_dataset.py     inputSTREAM.csv     (reach table)
  GIS/Limite.shp              ├──────────────────────────►    inputPONDS.csv      (pond table)
  GIS/demfill, GIS/lm_dem     │   geopandas + rasterio,       inputSTREAMw.asc    (renamed)
  GIS/202109MonitPts.shp     ─┘   run ONCE, by hand           inputSTREAMhmax.asc (renamed)
                                                              inputDEMfill.asc    (CRR + SFR routing)

### Steps

| # | Action | Files | Done when |
| --- | --- | --- | --- |
| 1.1 | Write code/tools/gis_to_dataset.py: reads the shapefiles/rasters named in [paths] data_root, writes the six intermediates above, prints a provenance header (source file, mtime, CRS, feature count) into each output. | new file | re-running it is idempotent; outputs carry their provenance |
| 1.2 | Rename the misnomer rasters inputPONDw.asc → inputSTREAMw.asc and inputPONDhmax.asc → inputSTREAMhmax.asc, with a compatibility shim that reads either name for one release. | example/LaMata/, marmites_sfr.py, marmites_postprocess.py | no code refers to a “pond” file that holds a stream |
| 1.3 | Retire the IN_007_gridSsurfhmax / IN_008_gridSsurfw input maps once SFR reads the reach table; replace them with an SFR-network map and a LAK-configuration map (a second mode of _fig_general_map, as flagged in the handoff). | marmites_postprocess.py | the input map set describes what the model actually uses |
| 1.4 | Point [paths] nwt_reference at the archive E:\00code_ws\LaMata_new_PhD_artigo_2s3L\_h5_MM.h5 instead of the repository dataset; keep the digest cache. | code/tests/plot_water_budget.py | the NWT comparison figures draw without a 1.3 GB copy in the repo |
| 1.5 | Write example/LaMata/README.md — the Tier A table of §6.1, one line per file: what reads it, what produced it, units. | new file | a fresh clone can be understood without archaeology |
| 1.6 | Add a pytest check that no file matching the excluded patterns (*.shp, *.tif, *.h5, output folders) exists under the repository. | code/tests/test_repo_hygiene.py | rule 1 is enforced by the suite, not by memory |
| 1.7 | Enumerate every parameter of the three legacy ini files (§6.4) into the WP0 schema — that is the authoritative definition of “the inputs MM needs”. __inputMM_v3.ini and __inputMMsurf.ini migrate parameter for parameter; __inputMF_flopy_v3_2s3L.ini is obsolete (MODFLOW-NWT) and is re-expressed for MF6, not carried across. | code/marmites_config.py, code/tools/flags_to_toml.py | every ini parameter has a schema key or is explicitly recorded as retired |
| 1.8 | Hand the enumeration to WP1b so the front-end presents all of it already filled from the current case study — nothing left for the user to discover. | code/app/pages/2_Configuration.py | opening the app on example/LaMata shows every parameter populated |

### Acceptance

A fresh clone plus DATA_ROOT reproduces a run; nothing geospatial is imported on the model path.

Every new repository file is one MM or MF reads directly, and is listed in the dataset README.

## WP1b — Streamlit web interface   [DONE, stage 1]

Objective. A browser front-end that browses the input data — the shapefiles included — edits and validates the run configuration, launches a run on this machine or on the server, follows it while it runs, and presents the output. It is a viewer and a launcher. It never becomes part of the model path.

### Why it belongs here, and why it is not one delivery

It sits on WP0 and WP1: it edits the configuration file WP0 defines and browses the data WP1 sorts into tiers. It is also the natural home for the geospatial libraries — decision D3 pushes geopandas and rasterio off the model path, and this is the one place they are welcome, alongside the WP1 converter. But its output side can only show figures that exist, so it is delivered in three stages:

| Stage | Delivered with | Content |
| --- | --- | --- |
| 1 | WP1 | Inputs page (Tier A tables + a real map of the Tier B shapefiles and rasters), Configuration page, Run page (launch + follow), a first Results page over the figures the native suite already writes |
| 2 | WP6 | Results grows with the new material: streamflow hydrographs, per-pond lake stage and volume, CRR routing, the ET split, soil moisture at depth against the probes |
| 3 | WP7 | Calibration page: phi progress, parameter and observation ensembles, per-group fits |

### Layout

app/                                REPO -- code only, as everything else in it
    Home.py                           entry point; the run registry and the machine banner
    pages/1_Inputs.py                 Tier A tables + Tier B shapefile / raster map
    pages/2_Configuration.py          read, edit and VALIDATE configs/*.toml
    pages/3_Run.py                    launch (local | server) and follow the log
    pages/4_Results.py                figures, water balance, hydrographs, obs fits
    pages/5_Calibration.py            pestpp-ies output                       (stage 3)
    lib/loaders.py                    cached readers, keyed on (path, mtime, size)
    lib/runs.py                       the run registry: launch, status, log tail
  .streamlit/config.toml              theme, server address and port, upload cap

### Steps

| # | Action | Files | Done when |
| --- | --- | --- | --- |
| 1b.1 | Scaffold the multi-page app and pin its dependencies in a separate optional group (streamlit, streamlit-folium or pydeck, geopandas, rasterio) so the model environment is untouched. | code/app/, environment-ui.yml | streamlit run code/app/Home.py opens; the model env still runs headless without any of it |
| 1b.2 | Inputs page. Tier A: each file as a table or a raster preview, with the units and the “read by” column of §6.1. Tier B: the shapefiles (hydrography, ponds, boundary, monitoring points, irrigated fields, EC tower) and the DEM on one map, layer toggles, click for attributes. | code/app/pages/1_Inputs.py, code/app/lib/loaders.py | every file listed in §6 is reachable from the page |
| 1b.3 | CRS handling. La Mata's projected CRS must be detected and reprojected to EPSG:4326 for a web map — reuse the CdL report_crs / reprojection pattern rather than assuming a CRS. Show the detected CRS in the sidebar. | code/app/lib/loaders.py | layers overlay correctly; a layer with a missing .prj is reported, not silently misplaced |
| 1b.4 | Caching. @st.cache_data on every reader, keyed on path + mtime + size (the same digest idea as the cbc cache); @st.cache_resource for the grid and open HDF5 handles. Without this, Streamlit re-reads a 180 MB coupled HDF5 on every widget click — it reruns the whole script each interaction. | code/app/lib/loaders.py | a widget click is instant on a full run |
| 1b.5 | Configuration page built from the MMConfig schema, not hand-written widgets, so new keys appear by themselves. Save refuses on a validation failure — the same fail-fast rule as WP0.6. Writes to code/configs/; never mutates a run's resolved_config.toml. | code/app/pages/2_Configuration.py | an invalid value cannot be saved; a new schema key appears unaided |
| 1b.6 | Run page — local or server. Launch the entry point as a detached subprocess writing <ws_root>/runs/<stamp>/run.log and status.json; the page polls the log tail. Never run the model in-line: a coupled run is ~11.5 min and an IES run is hours, and an in-line call would block the app and die on a browser refresh. | code/app/lib/runs.py, code/app/pages/3_Run.py | a run survives closing and reopening the browser; the log follows live |
| 1b.7 | Server mode. Two honest options; take the first: run Streamlit on the server (streamlit run code/app/Home.py --server.address 0.0.0.0 --server.port 8501) and reach it over the LAN — the PEST workers already run there, and the app only needs to see $MARMITES_WS_ROOT. The alternative, submitting over SSH from a local app, adds a credential path for no gain. | .streamlit/config.toml, docs | the same app serves both machines with one setting |
| 1b.8 | Results page — show, do not redraw. Present the PNGs run_postproc already wrote in out_<stamp>_<tag>/_output/, with a run picker and a figure filter. Add interactivity only where it buys something: a time slider on the head maps, a per-pond stage selector, an observation-point picker. Same rule as the native suite — one source of truth for a figure. | code/app/pages/4_Results.py | any archived run can be opened and compared without re-running anything |
| 1b.9 | Run the WP1 converter from the app: an “update dataset from GIS” action over code/tools/gis_to_dataset.py, with a preview of what would change before it writes. Makes the shapefile → plain-text step visible and repeatable instead of folklore. | code/app/pages/1_Inputs.py | the converter runs from the UI and reports its diff |
| 1b.10 | Keep it off the model path. code/app/ may import code/; code/ may never import code/app/ or streamlit. Add the assertion to the hygiene test. Bind to localhost by default. The launch action takes a validated configuration path only — never a free-text command box, since the page executes a process. | code/tests/test_repo_hygiene.py | the test fails if any code/ module imports streamlit |

### Acceptance

streamlit run code/app/Home.py on this machine and on the server serves the same app from one configuration setting.

Every input in §6 is inspectable from the browser, shapefiles on a map with their attributes.

A run is launched, followed and reviewed without touching a terminal — and the model still runs headless from Spyder and from a PEST worker with Streamlit not installed at all.

The one thing that would spoil this: the app quietly becoming a dependency of the model — a loader that only exists in code/app/lib, a configuration default that only the UI applies. The model must stay fully runnable from Spyder and from a PEST worker on a machine where Streamlit is not installed. Step 1b.10 is what keeps that true, and it is enforced by a test rather than by intent.

## WP1c — One grid path: Voronoi by default   [DONE, 1c.1-1c.8]

Objective. Move La Mata off its 50 m structured raster onto a Voronoi/DISV grid built the way CdL builds its own, so the two case studies share one discretisation and every surface package below is written once. The structured grid stays available as a producer, permanently.

Status 2026-09-10 - all eight steps implemented. A coupled La Mata run completes on a Voronoi mesh (ncpl 989, 469 active cells, cumulative discrepancy 0.000 % with SFR). The order of 1c.1/1c.2 was swapped and 1c.7 grew; see §11.5. Voronoi is not yet the default - [grid] kind stays "structured" until rung (c) of the ladder is judged on an equilibrated multi-year run. Modules built: code/marmites_meshes.py (producers), code/marmites_mesh.py (projection + resampling), code/ppMF6/marmites_topology.py (shared faces), code/ppMF6/marmites_rasterise.py (display adapter), code/tools/validate_grid.py (the ladder), code/app/pages/5_Grid.py (the mesh view).

### What already exists — more than expected

A survey of the code before planning this found the unstructured machinery largely present, and one piece of it genuinely clever:

| Layer | State | Note |
| --- | --- | --- |
| VertexGeometry | generic already | accepts arbitrary vertices/cell2d or a flopy VertexGrid. disv_from_structured is one producer, not an assumption |
| Cell list on an unstructured grid | exists | marmites_gridgen.refined_cell_list() returns (cid, cid, 0, icell2d) — setting i := cid, j := 0 so every legacy array[i, j] access resolves against (ncell, 1) column vectors. This is what lets raster-era MM code run on arbitrary cells |
| Raster → cell resampling | exists | resample_inputs + marmites_raster.sample_to_cells |
| RefinedModel | exists, tested | ties gridprops + active nodes + inputs + geometry together (test_refined_layout.py) |
| MF6 DISV build | exists | --grid disv |
| Coupler | ready | carries grid_kind and ncpl; cell area comes from geom |
| Driver wiring | missing | RefinedModel appears only in tests and a standalone quadtree script — it was never connected to the run |
| SFR routing | needs work | priority-flood over the fixed 8-neighbour raster stencil _NB8 |
| CRR neighbours | easier on Voronoi | CdL's shared-face _e2c edge→cells map is the reference |
| Post-processing + figures | raster-shaped | ~104 nrow/ncol references across marmites_postprocess.py and MARMITESplot_v3.py, no vertex branch. plotLAYER takes V shaped (ndays, nlay, nrow, ncol). This is the real cost of the package |

### Design — the grid becomes a pluggable producer

one function -> DISV gridprops + active_nodes, selected by  [grid] kind

    structured   today's raster re-expressed as DISV (disv_from_structured).
                 KEPT PERMANENTLY: the regression anchor and the MODFLOW-NWT comparison.
    voronoi      Triangle + flopy.utils.voronoi.VoronoiGrid, seeded as CdL seeds it:
                 graded stream corridor, centroid-seeded pond cells, background size.
                 CdL's section 3 transfers almost verbatim.          <-- THE DEFAULT
    quadtree     the existing gridgen path, kept because it already works.

  everything downstream consumes RefinedModel -> .cells  .geom  .inputs

### Steps

| # | Action | Files | Done when |
| --- | --- | --- | --- |
| 1c.1 | Wire RefinedModel into the driver — the missing link. [grid] kind selects the producer; the cached grid carries the configuration hash (WP0.6) so a stale mesh cannot be reused silently. | code/tests/run_lamata_mf6.py, marmites_gridgen.py | a run completes on a quadtree grid, proving the path end to end before Voronoi is added |
| 1c.2 | Add the voronoi producer: Triangle + VoronoiGrid, seeded from the watershed boundary, the stream lines and the pond polygons (the Tier A tables of WP1). Port CdL §3, including the Daoud-style graded transitions and the centroid pond seeding. | code/ppMF6/marmites_voronoi.py (new) | triangle.exe is at C:\00MODFLOW\win64\triangle.exe; the mesh builds and its ncpl is reported |
| 1c.3 | Upgrade the resampling. sample_to_cells samples at the cell centre; a 100 m Voronoi cell over a 50 m raster then throws away most of the data. Make it area-weighted for continuous fields and majority-vote for zone rasters, keeping centre sampling as an option. | code/marmites_raster.py | resampled means match the raster means within a stated tolerance |
| 1c.4 | Resample every La Mata input onto the mesh: soil zones, vegetation areas, soil thickness, ibound, elevation, hk, thickness, Sy/Ss, DRN and GHB. | marmites_gridgen.resample_inputs | the IN_* input maps redraw on the new grid and look right |
| 1c.5 | Shared-face neighbour topology from the cell2d edge lists (CdL's _e2c map). This replaces the fixed 8-neighbour stencil _NB8 in the SFR routing and is what CRR consumes in WP5. | code/ppMF6/marmites_topology.py (new), marmites_sfr.py | every interior face has exactly two cells; the graph is connected |
| 1c.6 | Resolve observation points by nearest centroid / point-in-polygon instead of (i, j). | marmites_postprocess.resolve_obs_cells | all 11 points resolve, and to the right cells |
| 1c.7 | Plotting adapter — do not rewrite the figure suite. Rasterise the per-cell result vectors onto a display raster and hand the native functions the (ndays, nlay, nrow, ncol) array they already expect. A thin adapter against ~104 raster references, and it keeps the whole validated suite including add_real_coord_axes. True-polygon maps come free in the Streamlit app (PlotMapView) and can be added natively later. | code/ppMF6/marmites_rasterise.py (new) | every figure redraws with no change to MARMITESplot_v3.py |
| 1c.8 | Validation ladder, extending test_disv_equivalence.py: (a) DIS vs DISV-from-DIS — must be identical, the existing Phase-4 baseline; (b) DISV-from-DIS vs a uniform-size Voronoi — water balance within a stated tolerance; (c) the production Voronoi against the validated 45-year DIS run — every difference attributed to discretisation, not to a bug. | code/tests/test_voronoi_grid.py (new) | the three rungs pass and (c) is written up |

### Acceptance

A full coupled La Mata run completes on a Voronoi grid, mass-conserving, with the whole figure suite drawn and no change to MARMITESplot_v3.py.

[grid] kind = "structured" still reproduces the pre-WP1c run exactly.

Accept this deliberately: La Mata's validated baseline — the RMSE, the saved hi_spinup_*.asc state, the MODFLOW-NWT comparison — is all on the structured grid. Voronoi invalidates the saved state, and the NWT comparison becomes a cross-grid comparison rather than like-for-like. That is precisely why the structured producer is kept rather than deleted: the regression stays runnable on the grid it was established on.

## WP2 — Total ET from three sources

Objective. Total ET = Ei + Eow + ETsoil (MM) + ETuzf (MF/UZF) + ETg (MM). Today MM supplies ETsoil and ETg and UZF does nothing (simulate_et=False), so the deep unsaturated zone evaporates nothing. Turn UZF's unsaturated ET on, keep its groundwater ET off, and make the demand accounting airtight so nothing is counted twice.

### 2a  The MF6 mechanism — verified, not assumed

MF6's UZF package lets ET be simulated in the UZF cell without simulating it in the GWF cell: switch SIMULATE_ET on, choose the unsaturated formulation (UNSAT_ETWC — the water-content form — or UNSAT_ETAE), and simply omit both LINEAR_GWET and SQUARE_GWET. Omitting them is what suppresses groundwater ET. This was confirmed against the flopy 3.10 UZF package definition shipped in the pinned environment, not from memory. The resulting call differs from CdL's in exactly one keyword:

CdL (all ET in UZF)                    MARMITES (unsaturated ET only)
  ───────────────────                    ──────────────────────────────
  simulate_et = True                     simulate_et  = True
  linear_gwet = True   ◄── ETg in MF     unsat_etwc   = True
  unsat_etwc  = True                     # linear_gwet / square_gwet  OMITTED  ◄── ETg stays in MM

### 2b  The demand chain

PET must be spent once. The proposed order per stress period, which follows the MARMITES formulation and keeps the residual explicit:

| Order | Consumer | Demand it sees | Where computed |
| --- | --- | --- | --- |
| 1 | Interception Ei | PE / PT partition from MMsurf | MM (MMsurf) |
| 2 | Open water Eow | Open water Eow — the wetted area of SFR reaches and LAK ponds. MMsurf computes the potential Eo and the coupler writes it to SFR and LAK SEPARATELY; what the chain subtracts is the ACTUAL evaporation read back from each, symmetric with ETuzf. LAK is kept apart from SFR throughout: how much a pond loses is a result in its own right. | MF (SFR, LAK) — demand from MMsurf, actual read back |
| 3 | Soil ETsoil = Σ(Esoil, Tsoil) over soil layers | what remains of PE and PT after 1–2 | MM (MMsoil) |
| 4 | Deep unsaturated ETuzf | the residual PET after 1–3, written to UZF PET per land-surface object; extdp = the rooting depth from land surface; extwc = θr | MF (UZF) — new |
| 5 | Groundwater ETg = Eg + Tg | the residual after 1–4 — computed from ETuzf ACTUAL, not from the PET written at step 4 (see below) — then limited by depth to water table | MM (MMsoil), applied as WEL Q |

The detail that makes step 5 correct — actual uptake, not demand: what we write to UZF is a demand; what UZF takes is bounded by water content — with UNSAT_ETWC it extracts at the PET rate only while θ > extwc, and nothing below it. In a semi-arid catchment the deep unsaturated zone is dry for much of the year, so actual uptake is often a small fraction of the demand. If step 5 subtracted the written demand, ETg would be starved precisely when the deep zone is dry — which is exactly when the deep-rooted oaks draw on the water table instead. The totals would still look plausible while the sourcing — the thing this model exists to compute — would be wrong. So: PET for ETg = PET_total − Ei − Eow − ETsoil − ETuzf_actual, read back from the UZF budget. This is what makes step 2.5 load-bearing rather than a reporting convenience.

Two consequences worth stating at the call site. First, simultaneity: ETg reaches MF6 as a WEL sink inside the same solve in which UZF computes its own ET, so within a stress period the two are simultaneous, not sequential. Strict sequential depletion is therefore exact only in iterative mode, where each outer iteration can use the current ETuzf iterate; lagged mode uses the previous stress period's actual. Second, depth overlap: UZF’s extdp is measured from the TOP OF THE MODFLOW MODEL, and here that top is the base of the MMsoil column — the driver sets top = land surface − soil thickness — so UZF’s column does not overlap the soil’s: it starts where the soil ends. extdp is therefore the part of each root zone below the soil column, max(Zr − soil thickness, 0), weighted by cover in each cell (et.extdp_from = vegetation). Corrected 2026-09-24: this paragraph first said to keep the full rooting depth, assuming UZF measured from the land surface; measured from the soil base, the full depth would reach Zr + soil thickness below ground. What prevents double counting of PET is still the demand chain — UZF can only take what remains of it.

Ordering caveat, state it in the code: in lagged mode MM cannot know step 4's outcome before MF6 solves, so ETg at step 5 uses the previous stress period's ETuzf actual. At daily stress periods that lag is small, but it is an approximation and must be documented at the call site and covered by a test, not discovered later.

### Steps

| # | Action | Files | Done when |
| --- | --- | --- | --- |
| 2.1 | Add [et] configuration: unsat_form (etwc¦etae), extdp (raster ¦ layer column ¦ value, the rule every spatial input follows), extwc_source. There is NO uzf_et switch and NO gwet_in_mf: UZF always simulates unsaturated-zone ET, and groundwater ET is never asked of MODFLOW at all. A switch forced to one position and read by nothing is worse than absent. | marmites_config.py, code/configs/lamata.toml | schema accepts and validates |
| 2.2 | Build the UZF package with simulate_et=True, the chosen unsaturated form, and no linear_gwet / square_gwet. Populate extdp, extwc, ha, hroot, rootact per land-surface object. Take extinction depth per vegetation zone (inputVEG*area.asc), as CdL does per COS class. Done 2026-09-24: et.extdp_from = vegetation, the cover-weighted root depth below the soil column; every UZF object of a column carries it. | code/ppMF6/marmites_mf6.py | the written .uzf shows SIMULATE_ET and no GWET keyword |
| 2.3 | Bind the UZF PET pointer in the coupler (same _bind_first size validation as SINF) and write the residual demand in the post-prepare_solve callback. | code/marmites_coupler.py | the log line coupler: UZF PET bound to … appears; PET varies per SP |
| 2.4 | Compute the residual demand in MMsoil: expose PET_residual after step 3, and accept an etuzf_actual_prev argument for step 5 — the actual uptake of the previous stress period, never the demand written at step 4. | code/MARMITESsoil/MARMITESsoil_v3.py | unit test on a single column reproduces the chain by hand, including the dry-deep-zone case where actual << demand |
| 2.5 | Load-bearing, not reporting: read ETuzf actual back from the UZF budget and feed it to step 5, as well as into wb_ts / wb_map. The code today harvests only UZF-GWRCH and REJ-INF, addressed by (text, paknam2); add the unsaturated-ET record and confirm its exact record text on the first run — the same discipline that uncovered the missing 1954-cell seepage face when both drain packages wrote under the text DRN. | marmites_coupler.py, marmites_postprocess.py | ETuzf actual reaches MMsoil and the water balance; the record text is pinned by a test |
| 2.5b | Emit a runtime PET-balance line per stress period — PET_total − Σ(Ei, Eow, ETsoil, ETuzf, ETg) — so starvation or over-extraction shows up in the log while a run is going, not in a test afterwards. | marmites_coupler.py | the line appears and stays at ~0 through a one-year run |
| 2.6 | GWET guard, the sibling of the infiltration-fidelity guard: fail the run if any groundwater-ET term in the UZF budget is non-zero. Now a REGRESSION guard rather than a misconfiguration one -- no configuration key can ask for linear_gwet or square_gwet any more -- so the test forces the keyword at the flopy call instead. | marmites_coupler.check_solution | a deliberately mis-set linear_gwet=True fails the run |
| 2.7 | New flux indices iETuzf and iETtot; add them to INDEX_MM, to flxIndex_lst, to the Sankey (a new UZF-ET arm) and to the catchment/obs time series. | code/marmites_indices.py, MARMITESplot_v3.py, marmites_postprocess.py | the Sankey closes with the new arm; MM-side mass balance still ~0 % |
| 2.8 | PET-closure test: per cell per stress period, Ei + Eow + ETsoil + ETuzf + ETg ≤ PE + PT + tol, and the run-total ET is reported split by source. | code/tests/test_et_partition.py (new) | test green on a one-year run |

### Acceptance

One-year coupled run: total ET splits into the five sources, the split sums to the demand, and the UZF groundwater-ET budget term is exactly zero.

Catchment-scale ET is compared against the MODFLOW-NWT reference and against the eddy-covariance record; any change from the current 0.7 % ETsoil agreement is explained by the new ETuzf arm, not by a shift in ETsoil.

Outcome (2026-09-25): accepted. The one-year coupled run (periodic spin-up, 2 cycles) splits total ET into its five sources, and no cell in no stress period has ETsoil + ETuzf + ETg above PE + PT — check_solution fails the run otherwise; what MF6’s UNSAT_ETWC removes beyond the demand within its own wave tolerance (1e-6 m per metre of unsaturated zone) is booked as UZF numerical loss, 0.33 m³ in the catchment-year. The UZF groundwater-ET term is zero. Catchment ET, mm/yr: Ei 33.6, ETsoil 182.0, ETuzf 12.4, ETg 33.0, against PE + PT 407.2 (44 % unmet). Against the MODFLOW-NWT reference over the same year ETsoil is 63 mm/yr lower, not the 0.7 % expected: 45.6 mm of it is the reference year drawing on its initial soil store (the periodic year draws none), the rest the vegetation cover now taken from the polygons (91 % grass against the legacy raster’s 25 %; interception 33.6 against 17.5) and the new UZF arm. Not done: the comparison with the eddy-covariance record — there is no actual-ET series in the dataset — and a run on the legacy vegetation raster to separate those causes, skipped by decision (2026-09-25).

## WP3 — SFR: run coupled, then validate

Objective. Take the existing, never-executed SFR build to a validated coupled run, sourcing the network from the hydrography shapefile rather than from the misnamed raster, and adopting the CdL reach rules.

Attribution, so the docs say the right thing: the CdL SFR build is not Daoud methodology. Daoud et al. 2022 is the source for the grid transitions (WP1c) and for CRR (WP5). CdL's SFR reads the stream lines from its GeoPackage, derives the ID/toID routing from the DEM, and gives each reach SFRmaker-style attributes (Leaf et al. 2021): one reach per cell, Manning rectangular, drainage-scaled width, downstream-monotonic bed. That is the method adopted here.

### 3a  Reach parameters: one source pattern, four producers

CdL computes width from drainage area and keeps Manning, streambed K and thickness uniform. This project generalises that: every reach parameter declares where its value comes from, and the same pattern serves LAK and CRR.

| Producer | Meaning | Typical use |
| --- | --- | --- |
| {value: 0.035} | one number for the whole network | Manning n, streambed thickness — the CdL default |
| {column: "rhk"} | from an attribute column of the stream shapefile | a surveyed or zoned streambed K |
| {raster: "..."} | sampled from a raster | a parameter that varies continuously |
| {drainage: {a: .., b: ..}} | computed from contributing drainage area, w = a·A^b (the arbolate-sum equivalent) | channel width — CdL's method, kept as an option and the default for width |

Where this is resolved — the point that keeps D3 intact: the converter (WP1.1) reads the shapefile, evaluates whichever producer each parameter declares, and bakes the per-reach number into inputSTREAM.csv. The Streamlit page (WP1b) is where the mapping is chosen and the converter is run. The model itself never opens a shapefile.

### Steps

| # | Action | Files | Done when |
| --- | --- | --- | --- |
| 3.1 | Source the network from inputSTREAM.csv (WP1) instead of inputSTREAMw.asc. Routing is derived from the DEM on the WP1c shared-face topology, not on the 8-neighbour raster stencil. | marmites_sfr.py | reach count and connectivity match the shapefile |
| 3.2 | Adopt the CdL reach rules: one reach per cell, Manning rectangular section, downstream-monotonic bed elevations. EVAPORATION = 0 stays — MM does E_ow. | marmites_sfr.py | no reach bed sits above the lowest bed upstream of it |
| 3.2b | Implement the parameter-source pattern of §3a, with drainage (w = a·A^b) kept as the default producer for channel width, and value for Manning, streambed K and thickness. Resolution happens in the converter, never in the model. | code/tools/gis_to_dataset.py, marmites_sfr.py, marmites_config.py | a width from drainage area and a width from a shapefile column both produce a valid network |
| 3.3 | Only the DRN cell that actually becomes the stream outlet is replaced by an outlet reach — the others are kept. CdL removes exactly one node's DRN-OUT record (the one that becomes the SFR inlet) and leaves the second outlet its drain. Replacing them all would delete legitimate boundary drainage; keeping the replaced one would drain the same water twice. | marmites_mf6.py | one drain record removed, the rest intact; outlet discharge appears once in the budget |
| 3.3b | Outlet-drain elevation = the BOTTOM of the layer, as in CdL — elev = botm[lay, cell], not a fixed depth below the cell top (CdL: drnw_records.append([(lay, c), float(botm[lay, c]), cond])). DRN-SEEP is a different package and stays at the land surface: it represents seepage where the water table reaches the ground, so its datum must not move. | marmites_mf6.py | outlet-drain elevations equal the layer bottoms; DRN-SEEP unchanged |
| 3.4 | Add an SFR continuous-observation block (inlet and outlet, per reach of interest) writing <model>.obs.sfr.csv, exactly as CdL does — WP7 consumes it. | marmites_mf6.py | the csv exists after a run, one column per observation |
| 3.5 | First coupled run — one hydrological year, sfr.enable = true, seep = drn, from the saved spin-up state. Watch: INFLOW binds (advanced-package variable — trap 5); the written reach inflow equals the MM runoff of channel cells; MVR balance; no non-convergence; discrepancy under the guard. | hand-off to Spyder | 0 failed time steps, mass balance < 0.1 % |
| 3.6 | Post-processing: streamflow hydrograph in mm/yr and m³/d at the outlet, against inputObsRo_catchment.txt; SFR budget figure. | marmites_postprocess.py | hydrograph drawn, obs overlaid |
| 3.7 | Full-period run and comparison of the catchment water balance with and without SFR. | hand-off | the difference is explained term by term |

Known physical limitation, document it once: an SFR reach exchanges directly with the aquifer — MF6 has no unsaturated zone beneath streams (MODFLOW-NWT's SFR2+UZF1 did; MF6 did not carry it over), and MVR cannot restore it because streambed leakage is an intrinsic package↔GWF connection, not water the package offers. In La Mata the channel follows the valley floor where the water table is shallow, so the bypass is acceptable — but say so in the model description rather than leaving it implicit.

## WP4 — LAK: ponds coupled, with the MM open-water exchange

Objective. Run the twelve EMBEDDEDV ponds coupled, wire the two missing MM↔LAK exchanges, and give the pre-processing the LAK configuration map that was asked for.

### 4a  Open water bypasses the MM soil column

User decision, 2026-09-09: wherever there is a LAK or SFR cell, the surface stands directly on UZF — the MM soil column is bypassed, so no ETsoil and no ETg there, only Eow, evaporation from open water.

On the Voronoi grid of WP1c this is applied wholesale, and it is exact, because the mesh is built so that the cell is the feature: every pond is centroid-seeded into its own pond-scale cell, so a LAK cell is a pond and nothing else. This is the case CdL was designed around, and it is what makes the decision clean rather than approximate — one more reason the grid change comes first.

Stream cells are the single place where the mesh cannot follow the feature: a channel is metres wide while a corridor cell is tens of metres, and meshing to channel width is not practical at catchment scale. The descriptor therefore carries a share, which is 1 for a pond cell by construction and the channel's share of the cell for a reach cell:

per cell:   f_lake + f_stream + f_soil = 1

    LAK cell   pond-scale, centroid-seeded    f_lake   = 1        wholesale bypass, exact
    SFR cell   corridor cell, channel in it   f_stream = w*L / A  the channel's share
    elsewhere                                 f_soil   = 1        the MM soil column, unchanged

  over f_lake + f_stream :  no ETsoil, no ETg, only Eow;  water goes to LAK / SFR
  over f_soil            :  the MM soil column, exactly as it behaves today

With EMBEDDEDV the lake exchanges directly with GWF, so the UZF column beneath a pond receives only the soil fraction's percolation — which is zero in a pond cell. Worth a comment at the call site.

“No ETg under open water” is not a loss of physics: groundwater discharge to the pond happens through the lakebed leakage term instead. Same flux, different name.

On the structured producer — kept only for regression — ponds are sub-grid and f_lake would fall well below 1. The descriptor covers that case automatically; it is simply no longer the production one.

WP4 and WP5 must share ONE mechanism: both perturb the same thing — which cells have a soil column, and in what proportion. Build a single per-cell surface descriptor (f_soil, f_lake, f_stream, summing to 1) once at build time, with the cell-list order untouched. Two independent surgeries on the Phase-2 cell-list contract is how that contract gets broken.

### Steps

| # | Action | Files | Done when |
| --- | --- | --- | --- |
| 4.1 | Source the ponds from inputPONDS.csv (WP1). Carry the CdL solver settings that stabilised the perched ponds: surfdep smoothing, maxiter = 200, stagechg = 1e-4; bed leakance from [lak] bedleak (1e-3 /d, the clay-lined charca value). | marmites_lak.py, marmites_mf6.py | package written, values traceable to the configuration |
| 4.2 | One EXTERNAL Manning spill outlet per lake — lakeout = -1, not 0 (trap 10). Equilibrium initial stage from the steady-state water table. | marmites_lak.py | no lake starts dry; spill routes to the downstream reach |
| 4.3 | Keep the MVR wiring for on-channel ponds: inlet reach → LAK, LAK spill → downstream reach. | marmites_mf6.py | diag-style mover listing shows both legs |
| 4.4 | New: bind LAK RUNOFF and write the MM runoff captured by each pond, in the post-prepare_solve callback. | marmites_coupler.py | pond inflow varies with MM runoff |
| 4.5 | New: read LAK STAGE back and give MMsoil the wetted area, so open-water evaporation uses the real pond area rather than a fixed footprint. LAK's own evaporation stays at zero. | marmites_coupler.py, MARMITESsoil_v3.py | E_ow tracks stage; no double evaporation |
| 4.6 | Build the per-cell surface descriptor of §4a — f_soil, f_lake, f_stream — once at build time, on the cell list as ordered. MMsoil applies its column to f_soil only; Eow acts on f_lake + f_stream; ETsoil and ETg are suppressed over the open fraction. Shared with WP5. | marmites_grid.py / the context builder, MARMITESsoil_v3.py | fractions sum to 1 per cell; a pond-scale Voronoi cell gives f_lake ≈ 1; the catchment water balance still closes |
| 4.7 | LAK configuration input map — a second mode of _fig_general_map, plus per-pond zoom pages in the CdL style (grid cells, SFR cells, LAK cells, footprint, stream). | marmites_postprocess.py | _input/IN_00x_lak_config.png and the zoom pages exist |
| 4.8 | Coupled run — one year with lak.enable = true on top of WP3, then the full period. | hand-off to Spyder | stages physically coherent, LAK budget closes, no non-convergence |

Expect low seepage and never-dry ponds. That was the CdL outcome at a bed leakance of 1e-3 /d and Alain confirmed it as the expected physics of clay-bottomed charcas. Do not treat it as a bug; it is the calibration lever, and in this project it becomes a PEST parameter (WP7).

## WP5 — CRR: the cascade-routing and reinfiltration module

Objective. Implement Daoud et al. (2022) Eq. 23 cascade routing with reinfiltration, in Python inside the coupler, with receiver priority LAK > SFR > downslope MARMITES soil column.

### The one structural risk

CRR makes runoff non-local: an upslope cell must be solved before the cell that receives its runoff, so cells must be visited in descending topographic order. The cell list is in build order, and both the coupler and the DISV mapping depend on that order. The change must therefore be introduced as a permutation computed once at build time and stored on the context (ctx.crr_order), never as a re-sort inside the stress-period loop. The cell list itself does not move. It shares the per-cell surface descriptor built in WP4.6 — one mechanism, not two.

### Steps

| # | Action | Files | Done when |
| --- | --- | --- | --- |
| 5.1 | Build the MFD weights once at build time on the WP1c shared-face topology: α_ij = β · S_ij / Σ S_ij over downslope neighbours; negative slopes drop out. Slopes from the sink-filled DEM sampled per cell. | code/ppMF6/marmites_crr.py (new), marmites_topology.py | weights sum to β per source cell |
| 5.2 | Compute and store ctx.crr_order — the descending-elevation permutation. Assert it is a permutation of the cell list and that every cell's receivers appear after it. | marmites_grid.py / the context builder | assertion holds on La Mata |
| 5.3 | Receiver resolution, priority LAK > SFR > downslope soil column; a cell with no downslope receiver is a topographic sink and its unrouted water evaporates (Daoud's convention). Count and report the sinks. | marmites_crr.py | sink count printed, matches the DEM |
| 5.4 | Reinfiltration enters the topmost MM soil layer, limited by Eq. 1b, I = min[Ssurf , D_soil,1 (φ1 − θ1)] — the same law as direct infiltration; the excess continues downslope. Do not confuse this with the rejected-infiltration return, which enters at the bottom and propagates upward and is already coded — see D4. | MARMITESsoil_v3.py | no cell exceeds its infiltration capacity; the bottom-up return still behaves exactly as before (regression test) |
| 5.5 | Deliver the residual reaching channel and pond cells to SFR INFLOW and LAK RUNOFF (WP3, WP4 wiring). | marmites_coupler.py | delivered volume equals the routed residual |
| 5.6 | Configuration: [crr] enable, beta (Daoud calibrates 0.8–1.0), sink_behaviour, order_source. enable = false restores today's behaviour exactly. | marmites_config.py | off-state is bit-identical to WP4 |
| 5.7 | Mass-balance test on a synthetic DEM (a 3×3 and a 5×5 with a known sink and a known outlet), then on La Mata: routed in = routed out + reinfiltrated + evaporated at sinks. | code/tests/test_crr.py (new) | closure to machine precision on the synthetic case |
| 5.8 | Coupled run with CRR on; compare the outlet hydrograph and the catchment balance against WP4. | hand-off | the difference is attributable to reinfiltration, and quantified |

Why not MVR: MVR moves water between MF6 packages, and MARMITES is not one. Routing the cascade through MVR would make the reinfiltration receiver a UZF cell — beneath the soil column — silently bypassing Ssoil, Esoil, Tsoil and Eq. 1b. Same physics and same parameters as CdL; only the receiver differs.

## WP6 — Post-processing and the observation exports

Objective. Extend the native suite to the new fluxes, port the CdL figures that have no MARMITES equivalent, and emit the four observation files WP7 consumes.

| # | Action | Files | Done when |
| --- | --- | --- | --- |
| 6.1 | Native suite: add ETuzf and total ET to the Sankeys, the catchment and per-point time series and the flux maps; add SFR and LAK arms. | MARMITESplot_v3.py, marmites_postprocess.py | one --postproc-only --preproc emits the lot, no skips |
| 6.2 | Port from postprocess_cdl.py: streamflow hydrographs in mm/yr, lake stage and volume panels per pond, MVR accounting (per-mover budget), the water-balance graphs. | marmites_postprocess.py | per-pond panels drawn for all ponds |
| 6.3 | Soil-moisture-at-depth figure and export — MM_S[:, :, iSsoil_pc_s] per soil layer at each observation point, against inputObsSM_*. This is the new calibration target. | marmites_postprocess.py | one page per observation point, one curve per depth |
| 6.4 | Observation exports (the WP7 contract), written on every run: obs_heads.csv, obs_sm.csv, obs_sfr.csv, obs_et.csv — long format, period-end rows, deterministic row count. | marmites_postprocess.py | row counts equal nper regardless of ATS sub-stepping |
| 6.5 | Extend plotCALIBCRIT to the new groups (streamflow, ET) alongside heads and soil moisture. | MARMITESplot_v3.py | NSE / RMSE / RSR / r reported per group |
| 6.6 | WP1b stage 2 — surface all of the above on the Results page: streamflow, per-pond stage and volume, CRR routing, the ET split, soil moisture at depth. Show the written figures; add a run picker and a run-to-run comparison. | code/app/pages/4_Results.py | a run is reviewed end to end from the browser |

ATS determinism — the trap CdL already hit: adaptive time stepping gives a parameter-dependent number of rows in a time-series output, which breaks the positional PEST instruction files. Collapse every exported series to stress-period-end rows (CdL's obs_collapse.py) in the same way in the reference run and in every forward run.

## WP7 — PEST++-IES calibration

Objective. Calibrate the coupled MM+MF6 model against four observation groups — heads, soil moisture at depth, streamflow, total ET — with pyEMU and pestpp-ies, reusing the CdL calibration architecture and, more important, its hard-won failure modes.

The design is in §8. The steps here are the build order.

| # | Action | Files | Done when |
| --- | --- | --- | --- |
| 7.1 | Externalise the model inputs so pyEMU can template them: MF6 side via sim.set_all_data_external(); MM side by writing the soil/vegetation parameter maps and inputSOILparam.txt as flat, one-value-per-line files (pyEMU reads with np.loadtxt, which needs a rectangular file — CdL hit exactly this on wrapped DISV arrays). | code/tools/pest_prep_mm.py (new) | org/ builds and MF6 runs from it unchanged |
| 7.2 | Build the PEST interface with PstFrom: pilot points for Kh per layer with a geostatistical structure, constants/multipliers for the boundary conditions, and the MM soil / vegetation / ET parameters (§8.1). | code/tools/build_pst_mm.py (new) | mm.pst builds; parameter count reported |
| 7.3 | Register the four observation groups from the WP6 exports, with the weighting scheme of §8.2. | same | pst.obs_groups = heads, sm, sfr, et |
| 7.4 | Forward run — the piece that differs most from CdL. forward_run_mm.py must: apply the multipliers; run the coupled MM+MF6 driver through libmf6 (not mf6.exe); collapse the outputs to period-end rows; write the four observation csvs. Guard it with multiprocessing.freeze_support() — without it the apply step silently kills the worker. | code/tools/forward_run_mm.py (new) | a base run (noptmax = 0) reproduces the reference phi |
| 7.5 | Draw the geostatistical prior ensemble (prior_pe.jcb) and build the first-order Tikhonov smoothness equations (prior_cov.jcb). Not optional — see the callout. | code/tools/build_pst_mm.py | prior drawn; PI equation count in the low thousands |
| 7.6 | Prior Monte-Carlo (noptmax = -1) to check the machinery and see the prior spread, then the base run, then the IES iterations. | code/tools/run_ies_mm.py (new) | prior MC completes; failure rate < 30 % |
| 7.7 | Post-process: phi progress, parameter and observation ensembles, per-group fits, and the soil-moisture-at-depth fit. | code/tools/postproc_ies_mm.py (new) | figures written to ies_output/ |
| 7.8 | Extract the minimum-phi realisation into pest_optimised.npz and add a [pest] use_pest_params switch so a normal run can use the calibrated set. | code/tools/extract_pest_optimised_mm.py, marmites_config.py | a calibrated run reproduces the IES base |
| 7.9 | WP1b stage 3 — the Calibration page: phi progress, parameter and observation ensembles, per-group fits, and the launch button for run_ies_mm.py on the server (detached, like any other run, but measured in hours). | code/app/pages/5_Calibration.py | an IES run is started and followed from the browser |

Carry the CdL scars into the first build, not the second: the first CdL IES overfitted to a checkerboard K field — pestpp draws its prior from a diagonal, bounds-only covariance, so pilot points are spatially white. The fix package was: a geostatistical prior ensemble, first-order Tikhonov smoothness at ies_reg_factor ≈ 0.5, moderated bounds (wide enough to explore, tame enough that most realisations converge — ultra-wide bounds bought 60 % failures and a biased survivor set), ies_bad_phi_sigma = 2.5, and an absolute overdue_giveup_minutes cap. Build all of it in from the start.

## WP8 — Tests, documentation and merge hygiene

| # | Action | Done when |
| --- | --- | --- |
| 8.1 | New test modules: test_config.py (extended), test_et_partition.py, test_sfr_coupled.py, test_lak_coupled.py, test_crr.py, test_repo_hygiene.py (repo rule and the “code/ never imports streamlit” assertion). | suite green, no new skips |
| 8.2 | Run the six mf6-binary tests with E:\00code\moflowapi\emsdatasets\bin on PATH — their fixture was repaired on 2026-09-09 but has never been executed. | 6 skips become 6 passes |
| 8.3 | Rewrite MARMITES_NEXT_STEPS.md §0-ter (flags → configuration) and §4 (SFR/LAK/CRR status); record the WP4.6 and WP2 ordering decisions in the analysis document. | docs match the code |
| 8.4 | Clean the pre-existing cosmetics: unused locals and imports in run_lamata_mf6.py (NCROP, check, the re-imported flopy) and two in marmites_postprocess.py. | lint clean |
| 8.5 | Plan the MM-MF6 → master merge: 7 modify/delete conflicts on trunk/pyEARTH1D/* (that path exists on master) (master modified them; the Phase-1 cleanup deleted them here). Decide whether pyEARTH1D stays on master before merging, not during. | decision recorded; merge rehearsed on a scratch branch |
| 8.6 | Report the flopy get_structured_faceflows(ia=, ja=) defect upstream — it has been verified on 3.10 and 3.11 and no issue is filed. | issue opened |

## WP9 — Port to CdL, the production case study

Objective. Make Casa de Lobos the second case study of the same code, populating example/CdL/ rather than writing a second model. WP1c has already removed the grid obstacle; what remains is mostly data.

Step 9.1 is a data question, not a coding one — and it gates the rest: CdL carries monthly precipitation and ET0. MMsurf needs hourly meteorology (rainfall, wind speed, air relative humidity, temperature, incoming solar radiation) to produce the daily series MMsoil consumes. Until that record is found or assembled, MARMITES cannot run on CdL at any grid resolution. Answer this before scheduling the rest of the package.

| # | Action | Done when |
| --- | --- | --- |
| 9.1 | Source the daily/hourly meteorology. Options in order of preference: an existing station record for the catchment; a national network series (IPMA) disaggregated; or a reanalysis product downscaled. Whatever is chosen, MMsurf must be able to run from it and produce the inputZON*_d.txt series. | MMsurf runs for CdL and writes the daily series |
| 9.2 | Soil and vegetation zoning in the MARMITES sense. Derive MM soil zones from the CdL soil hydraulic rasters (ks, ths, wp, fc) and vegetation areas from COS-2025 — the same rasters the CdL model already samples per cell. Write inputSOILparam.txt and the .asc zone maps. | the IN_* parameter maps draw for CdL and are physically sensible |
| 9.3 | Grid: reuse CdL's own Voronoi mesh through the WP1c voronoi producer, or regenerate it from the same seeds. The voronoi_layers.npz layering (U1/U2/U3) maps onto the MM layer handling. | the mesh loads and ncpl matches the CdL model |
| 9.4 | Observations into the MARMITES formats: inputObs.txt plus the per-point head, soil-moisture and streamflow files. Note that CdL's piezometers are synthetic/regionalised, so weight them accordingly in WP7's scheme. | the observation exports populate for CdL |
| 9.5 | SFR / LAK / CRR from the CdL geometry — the streams and the 19 charcas, through the same converter and the same parameter-source pattern as La Mata. Nothing case-specific in code/. | a CdL run builds with the surface packages on |
| 9.6 | Run, validate, then calibrate, reusing the WP7 chain unchanged. Compare against the existing MF6-only CdL model as a cross-check: the aquifer side should be recognisable. | a coupled CdL run converges and its water balance is defensible |

Estimate: dominated by 9.1 and 9.2, which are data-assembly tasks rather than programming ones. The code work (9.3, 9.5) should be small precisely because WP0, WP1 and WP1c were done first.

# 6  Input-data inventory

Two tiers. Tier A lives in the repository because MM or MF reads it directly; Tier B lives in DATA_ROOT outside the repository and is consumed only by the one-off converter, by the pre-processing figures, or by the calibration.

## 6.1  Tier A — in the repository (example/<case>/, e.g. example/LaMata/)

| Group | File(s) | Content / units | Read by |
| --- | --- | --- | --- |
| Run control | __inputMM_v3.ini → code/configs/lamata.toml | run and model settings (WP0) | run_lamata_mf6.py |
| Time | inputDATE.txt | the stress-period calendar | MM, post-processing |
| Meteo / vegetation series | inputZON*_d.txt, inputZON_*_stp.txt (P, Pe, RF, RFe, TF, PT, PE, E0, Eo, LAI, crop) | per zone, daily and per stress period, mm/d | MMsurf → MMsoil |
| Zoning maps | inputMETEOzones.asc, inputIRRzones.asc, inputSOILzones.asc, inputVEG{1,2,3}area.asc | integer zone / fractional area rasters | MM |
| Soil parameters | inputSOILthick.asc, inputSOILparam.txt | soil thickness (m); per-zone θs, θr, θfc, Ks, layer split | MMsoil |
| Aquifer grids | MF_ws/elev.asc, elev_sinkfil.ASC, ibound_l*.asc, hk_l*.asc, thick_l*.asc, Sy_l*.asc, Ss_l*.asc, uzf_iuzfbnd.asc | per-layer aquifer properties | marmites_mf6.py |
| Boundary conditions | MF_ws/drn_cond_l*.asc, drn_elev_l*.asc, ghb_cond_l*.asc, ghb_head_l*.asc | DRN and GHB geometry and conductance | marmites_mf6.py |
| Saved state | MF_ws/hi_spinup_l*.asc, hi_spinup_perc.asc, hi_spinup_etg.asc | spin-up heads and mean recharge / ETg (baseline fallback) | marmites_mf6.py, coupler |
| MF6 parameter sets | MF_ws/__inputMF_flopy_v3_2s1L.ini, _2s3L.ini | 2- and 6-layer parameter sets | marmites_mf6.py |
| Observations — heads | inputObs.txt, inputObsHEADS_{C1..C5,P0,W1}.txt | point name / X / Y / layer; date, head (m) | post-processing, PEST |
| Observations — soil moisture | inputObsSM_P0.txt, inputObsSM_SM.txt | date, θ (%) per probe depth — the WP7 sm group | post-processing, PEST |
| Observations — runoff | inputObsRo_catchment.txt | catchment runoff / streamflow | post-processing, PEST |
| NEW — stream | inputSTREAM.csv | reach table: id, i/j (or node), toID, length, slope, width, bed top, Manning n, streambed K and thickness | marmites_sfr.py |
| NEW — stream rasters | inputSTREAMw.asc, inputSTREAMhmax.asc (renamed from inputPONDw / inputPONDhmax) | channel width (m) and depth (m) per cell | marmites_sfr.py, MM E_ow |
| NEW — ponds | inputPONDS.csv | pond table: fid, centroid X/Y, area (m²), rim elevation, bottom elevation, host cell, on-channel flag, inlet/outlet reach | marmites_lak.py |
| NEW — DEM | inputDEMfill.asc | sink-filled DEM (m) — CRR slopes and SFR routing | marmites_crr.py, marmites_sfr.py |
| NEW — grid seeds | inputWATERSHED.csv | catchment boundary polygon — the Voronoi domain (with the stream and pond tables above, these are the three mesh seeds) | marmites_voronoi.py (WP1c) |
| NEW — ET | inputEXTDP.asc (or a per-vegetation-zone table) | ET extinction / rooting depth (m) per cell — the UZF extdp column | marmites_mf6.py |
| NEW — ET observations | inputObsET_catchment.txt (or per zone) | observed actual ET (mm/d or mm/month) — the WP7 et group | post-processing, PEST |

## 6.2  Tier B — outside the repository (DATA_ROOT)

| Group | Source (current location) | Consumer | Why it stays out |
| --- | --- | --- | --- |
| Stream network | E:\00code_ws\LAMATA_new\GIS\hydrography.shp (also Streams.shp, streams_dem.shp, stream_buffer.shp) | code/tools/gis_to_dataset.py → inputSTREAM.csv | shapefile; needs geopandas |
| Ponds | GIS\lm_ponds.shp | → inputPONDS.csv | same |
| Catchment boundary | GIS\Limite.shp | the general map, the converter | same |
| DEM | GIS\lm_dem, GIS\demfill, GIS\lm_streamord | → inputDEMfill.asc; hillshade figures | raster; needs rasterio |
| Monitoring / observation points | GIS\202109MonitPts.shp, 202109ObsPts.shp, PilotsPoints.shp | the general map; PEST pilot-point layout | shapefile |
| Irrigated fields, EC tower | GIS\Irr_Fields.shp, ECtower_footprint.shp | the general map; the ET observation footprint | shapefile |
| MMsurf inputs and outputs | E:\00code_ws\LaMata_MM-MF6\MMsurf_ws, and the provenance copy in LaMata_new_PhD_artigo_2s3L\MMsurf_ws | MMsurf (py3-ported, currently wired into nothing) | run output / provenance |
| MODFLOW-NWT reference | E:\00code_ws\LaMata_new_PhD_artigo_2s3L\_h5_MM.h5 (1.3 GB), MF_ws\_h5_MF.h5 (3.1 GB) | plot_water_budget.load_reference() — the NWT-vs-MF6 panels | 1.3 + 3.1 GB, cannot be regenerated |
| All run output | $MARMITES_WS_ROOT = E:\00code_ws\LaMata_MM-MF6 (MF6_ws/, out_<stamp>_<tag>/) | everything | rule 1 |
| PEST workspace | $MARMITES_WS_ROOT\MM_pest (org/, template/, master/, workers/) | pyEMU, pestpp-ies | generated; large; per machine |
| Binaries | C:\00MODFLOW\mf6.7.0_win64\bin\libmf6.dll and mf6.exe; the PEST++ suite | the coupler, the calibration | machine-specific — mm_paths.py |

Every shapefile and raster in this table is also a layer on the WP1b Inputs page — that page and the WP1 converter are the only two consumers of Tier B, and the only two places geopandas and rasterio are imported.

## 6.3  Data that does not exist yet and must be assembled

| Item | What is needed | Candidate source |
| --- | --- | --- |
| Observed actual ET | a catchment or zonal actual-ET series to weigh the et group against | the eddy-covariance tower already in the GIS (ECtower_footprint.shp) — its flux record; failing that, MODIS MOD16A2GF zonal aggregation, exactly as CdL does (download_mod16.py, make_mod16_zonal.py) |
| Streamflow at the outlet | a gauged or regionalised series for the sfr group | inputObsRo_catchment.txt if it is a discharge; otherwise the CdL donor-gauge regionalisation method |
| Soil-moisture probe depths | the depth of each probe, to map it onto an MM soil layer | inputObsSM_* headers plus the field notes — must be pinned before WP7.3 |
| Streambed conductance | reach-level streambed K and thickness | not in the dataset today (the six DRN cells carry conductance 0.025–0.035). Start from the CdL value and make it a PEST parameter |
| Pond bathymetry | pond depth for the stage–volume table | POND_DEPTH fallback (CdL uses 4.0 m below local ground); refine from survey if it exists |

## 6.4  The legacy ini files — the authoritative parameter list

The tables above name the data files. The parameters are defined by the three ini files in example/LaMata/, and those are what the WP0 schema must cover key for key and the WP1b front-end must present already filled. Nothing may be quietly dropped in the migration.

| File | Blocks it defines | Fate |
| --- | --- | --- |
| __inputMM_v3.ini (122 lines) | run control (run_name, verbosity, MMsurf_yn, MMsoil_yn, MF_yn, MF_lastrun); convergence loop (convcrit, convcritmax, ccnum); plotting (plt_out, plt_freq, nrangeMM/MF, ctrsMM/MF, ntick, animation, plt_out_obs, WBsankey_yn, plt_WB_unit, iniMonthHydroYear, tick-grid years, plt_input); workspaces and file names (MMsurf_ws, inputFile_PAR_fn, inputFile_TS_fn, MF_ws, MF_ini_fn, outputFILE_fn, MMsurf_fn); irrigation package (irr_yn, inputFile_TSirr_fn, gridIRR_fn); grid origin (xllcorner, yllcorner); the raster names (gridMETEO_fn, gridSOIL_fn, gridSOILthick_fn, gridPONDhmax, gridPONDw); soil parameters (SOILparam_fn); observations (inputObs_fn, inputObsHEADS_fn, inputObsSM_fn, inputObsRo_fn, rmseHEADSmax, rmseSMmax); HDF5 compression | migrates key for key into [run] [postproc] [paths] [irr] [grid] [obs]. gridPONDhmax / gridPONDw are the misnamed stream rasters (trap 12) and retire with SFR |
| __inputMMsurf.ini (99 lines) | meteo zones (NMETEO; per zone phi, Lm, Z, Lz, FC, DTS, z_m, z_h); vegetation (NVEG; per type h_d, h_w, S_w, C_leaf_star, LAI_d, LAI_w, f_s_vd, f_s_vw, alfa_vd, alfa_vw, J_vd, J_vw, TRANS_vdw, TRANS_vwd, Zr max root depth, and the transpiration-sourcing factors kTg_min, kTg_max, kT_f, kT_1/s); crops (NCRP, NFIELD, per crop h_c, S_w_c, C_leaf_star_c, LAI_c, f_s_c, alfa_c, Zr_c, its kT*_c); soils (NSOIL; per type por, fc, alfa_sd, alfa_sw, J_sd, J_sw, TRANS_sdw, TRANS_swd); open water (alfa_w) | migrates key for key into [mmsurf.meteo] [mmsurf.veg] [mmsurf.crop] [mmsurf.soil] [mmsurf.openwater]. Zr is also the natural source for the WP2 [et] extdp_source = "veg_zone" rooting depth |
| __inputMF_flopy_v3_2s3L.ini | the MODFLOW-NWT model: nlay/nrow/ncol/nper, itmuni, lenuni, delr, delc, elev_fn, ibound_fn, hnoflo, the LPF/UPW blocks (laytyp, layavg, chani, layvka, laywet, vka_fn, hdry, iphdry…), OC and budget extensions | OBSOLETE — do not migrate. Written for MF-NWT. Its *content* (layer geometry, aquifer properties, boundary conditions) is re-expressed for MF6 in [layers] [uzf] [seep] and the MF_ws/*.asc grids; the NWT-only solver and package keys are recorded as retired, not carried across |

Also parameterised, and outside the ini files: MF_ws/inputSOILparam.txt — per soil zone, the number of soil layers nsl, and per layer the soil type st, thickness proportion slprop, Smax, Sfc, Sr, Si and Ks. That is the file the WP7 soil_* parameter group calibrates against, and it must be externalised in WP7.1.

# 7  Configuration reference

## 7.1  code/mm_paths.py — the machine layer

"""The only machine-specific paths. Edit here, or set the MM_* environment variables."""
import os
from pathlib import Path

REPO       = Path(__file__).resolve().parents[1]                     # the MARMITES checkout
DATA_ROOT  = Path(os.environ.get('MM_DATA_ROOT',  r'E:\00code_ws\LAMATA_new'))
WS_ROOT    = Path(os.environ.get('MM_WS_ROOT',    r'E:\00code_ws\LaMata_MM-MF6'))
NWT_REF    = Path(os.environ.get('MM_NWT_REF',    r'E:\00code_ws\LaMata_new_PhD_artigo_2s3L\_h5_MM.h5'))
MODFLOW_DIR= Path(os.environ.get('MM_MODFLOW_DIR',r'C:\00MODFLOW'))

LIBMF6     = str(MODFLOW_DIR / 'mf6.7.0_win64' / 'bin' / 'libmf6.dll')
MF6_EXE    = str(MODFLOW_DIR / 'mf6.7.0_win64' / 'bin' / 'mf6.exe')
PESTPP_IES = str(MODFLOW_DIR / 'pestpp' / 'bin' / 'pestpp-ies.exe')
PYTHON_EXE = os.environ.get('MM_PYTHON_EXE', r'C:\miniconda3\envs\flopy\python.exe')

CASE       = 'LaMata'                     # the case study
DATASET    = REPO / 'example' / CASE      # Tier A — read directly by MM/MF
GIS        = DATA_ROOT / 'GIS'            # Tier B — converter input only

Environment note: the flopy environment must be invoked by prefix — C:\miniconda3\envs\flopy\python.exe — because two identically-named environments exist on this machine and activating by name has crashed rasterio. PYTHON_EXE above is what the PEST workers will be given, so it must be the real one.

## 7.2  code/configs/lamata.toml — the model and run layer (skeleton)

[meta]
config_version = 1
name           = "lamata_2lay_sfr_lak_crr"    # -> out_<stamp>_<name>

[paths]
case      = "LaMata"                 # -> example/LaMata/, the Tier A inputs
ws        = ""                       # blank -> WS_ROOT/MF6_ws
gis_ws    = ""                       # blank -> DATA_ROOT/GIS (figures only, never the model path)

[run]
mode        = "lagged"      # lagged | iterative        (was --mode)
relax       = 0.6           # iterative under-relaxation (was --relax)
nsp         = 0             # 0 = all stress periods     (was --nsp)
daily       = true          # false = aggregated         (was --aggregated)
ats         = true          #                            (was --no-ats)
build_only  = false         #                            (was --build-only)
max_discrepancy = 1.0       #                            (was --max-discrepancy)

[grid]                      # ---- WP1c ----
kind = "voronoi"            # voronoi (DEFAULT) | structured | quadtree
rebuild = false             # true = re-mesh; the cache carries the config hash
  [grid.voronoi]
  cell_far        = 100.0   # m, background cell size
  cell_near_stream = 40.0   # m, stream corridor
  stream_buffer   = 60.0    # m
  stream_refine   = true    # false = SFRmaker style, network mapped onto the background
  trans_levels    = [10.0, 20.0, 40.0, 70.0]   # Daoud-style graded transitions
  seed_ponds      = true    # one pond-scale cell per pond, centroid-seeded

[layers]
nlay      = 2               # 2 reads _2s1L.ini directly (was --nlay)
aggregate = false           # 6->2 derivation, comparison only (was --aggregate)

[uzf]
vks_scale = 1.0             #                            (was --uzf-vks-scale)

[seep]
kind = "drn"                # uzf | drn                 (was --seep)
cond = 10000.0              # must be free-draining      (was --seep-cond)

[et]                        # ---- WP2, all new ----
uzf_et       = true         # UZF SIMULATE_ET on
unsat_form   = "etwc"       # etwc | etae
gwet_in_mf   = false        # MUST stay false: ETg comes from MM (guarded)
extdp_source = "veg_zone"   # uniform | veg_zone | raster
extdp_default = 2.0         # m, fallback rooting depth

[sfr]                       # ---- WP3 ----
enable = true               #                            (was --sfr)
min_slope = 1e-4
source = "inputSTREAM.csv"  # table from tools/gis_to_dataset.py
monotonic_bed = true        # SFRmaker rule (Leaf et al. 2021)
# parameter-source pattern (section 3a): each parameter says WHERE its value
# comes from -- value | column | raster | drainage. Resolved by the CONVERTER,
# never by the model, so no shapefile is ever on the model path.
  [sfr.width]
  drainage = { a = 0.5, b = 0.35 }  # w = a*A^b, A = drainage area -- CdL's method, THE DEFAULT
  # column = "width_m"              # or: an attribute column of the stream shapefile
  # value  = 2.0                    # or: uniform over the network
  [sfr.manning]
  value = 0.035
  [sfr.rhk]
  value = 0.1               # m/d streambed K            (was --sfr-rhk)
  [sfr.rbth]
  value = 0.5               # m streambed thickness

[lak]                       # ---- WP4 ----
enable   = true             #                            (was --lak)
bedleak  = 1e-3             # 1/d                        (was --lak-bedleak)
surfdep  = 0.05             # m
maxiter  = 200              # LAK Newton cap
stagechg = 1e-4             # m
source   = "inputPONDS.csv"

[crr]                       # ---- WP5 ----
enable = true
beta   = 1.0                # Daoud Eq. 23 partitioning (calibrated 0.8-1.0)
sinks  = "evaporate"
dem    = "inputDEMfill.asc"

[spinup]
cycles     = 1              #                            (was --spinup)
tol        = 0.05           #                            (was --spinup-tol)
strt_heads = "hi_spinup"    #                            (was --strt-heads)
steady_means = "hi_spinup"  #                            (was --steady-means)
strt_dem   = []             # [a, b] regression          (was --strt-dem)

[postproc]
enable = true               #                            (was --postproc)
preproc = true              #                            (was --preproc)
only    = false             #                            (was --postproc-only)
sankey_min_flux = 0.05      #                            (was --sankey-min-flux)
sankey_full = true          #                            (was --no-sankey-full)
map_days = 6                #                            (was --map-days)

[pest]                      # ---- WP7 ----
use_pest_params = false
params_npz      = "pest_optimised.npz"

[ui]                        # ---- WP1b, read ONLY by app/, never by the model ----
execution = "local"         # local | server  -- where the Run page launches the model
runs_dir  = ""              # blank -> WS_ROOT/runs   (log + status.json per launch)
port      = 8501
address   = "localhost"     # "0.0.0.0" when serving from the server over the LAN
poll_secs = 3               # log-tail refresh interval on the Run page

## 7.3  Rules the configuration layer must enforce

Unknown keys raise. A typo must not fall through to a default.

The resolved configuration is written into the run folder and hashed into the coupled HDF5.

Caches carry the hash they were built from; a mismatch stops the run naming the offending key. The CdL grid-design cache silently overrode a user setting once — that is the whole reason for this rule.

--set section.key=value is the only override, and every override is echoed at startup.

# 8  PEST-IES calibration design

## 8.1  Parameters

| Group | Parameter | Style / bounds | Notes |
| --- | --- | --- | --- |
| kh{1,2} | horizontal K per layer, pilot points | multiplier, 0.2 – 5 | geostatistical structure per layer; range ≈ 4 × pilot-point spacing. Kv follows Kh at a fixed ratio in the forward run |
| sy, ss | specific yield / storage per layer | multiplier, 0.2 – 5 | one constant per layer to start |
| drn_cond | outlet drain conductance | multiplier, 0.05 – 20 | split by layer if the two-unit behaviour repeats what CdL saw |
| drnseep_cond | surface seepage-drain conductance | multiplier, 0.05 – 20 | the baseflow lever; keep it free-draining |
| ghb_cond, ghb_head | lateral boundary | multiplier 0.05 – 20 / additive ±5 m | if GHB is retained |
| sfr_rhk | streambed K | multiplier, 0.05 – 20 | no field value exists — see §6.3 |
| lak_bedleak | lakebed leakance | multiplier, 0.1 – 10 | the NbS recharge lever; physical range 7e-4 – 1.5e-3 /d |
| crr_beta | cascade partitioning β | value, 0.5 – 1.0 | Daoud calibrates 0.8 – 1.0 |
| soil_* | per soil zone and layer: θs, θr, θfc, Ks, soil thickness | multiplier, moderate bounds | what the soil-moisture group actually informs |
| et_extdp | rooting / extinction depth per vegetation zone | multiplier, 0.5 – 2 | splits ETuzf against ETg — the ET group's main lever |
| et_partition | PE / PT split, interception coefficient | multiplier, narrow | only if the ET residuals demand it; correlated with et_extdp |

## 8.2  Observations

| Group | Source file (WP6 export) | Content | Weighting |
| --- | --- | --- | --- |
| head | obs_heads.csv | 11 monitoring points, period-end heads | 1/σ with σ from the measurement record; then balanced against the other groups |
| sm | obs_sm.csv | θ per probe depth at the points that have probes — from MM_S[:,:,iSsoil_pc_s] against inputObsSM_* | the new group; weight so it carries a share comparable to head |
| sfr | obs_sfr.csv | outlet streamflow (and inlet, if one is prescribed), period-end | relative weighting on the log of discharge, so low flows are not drowned by peaks |
| et | obs_et.csv | total ET = Ei + Eow + ETsoil + ETuzf + ETg, catchment or zonal, monthly | against the EC tower or a MODIS zonal aggregation (§6.3) |

Start with group-balanced weights — each group contributing an equal share of the initial phi — and adjust only with a reason. This is where the model is most easily fooled: heads alone will happily be fitted by a checkerboard K field, and it takes streamflow, soil moisture and ET together to constrain the partitioning that MARMITES exists to compute.

## 8.3  The forward run

forward_run_mm.py  (executed in every PANTHER worker directory)
  ───────────────────────────────────────────────────────────────
   1. pyemu.helpers.apply_list_and_array_pars(...)        # multipliers -> MF6 + MM input files
   2. derive Kv = KV_RATIO * Kh for every layer file
   3. run the COUPLED driver: MM + MF6 through libmf6     # NOT mf6.exe -- this is the CdL difference
   4. collapse every time-series output to period-end rows (ATS determinism)
   5. write obs_heads.csv / obs_sm.csv / obs_sfr.csv / obs_et.csv
  guarded by  if __name__ == '__main__': multiprocessing.freeze_support()

## 8.4  Run-time budget — decide this before building the interface

A full La Mata run is 1949 daily stress periods and takes roughly 11–12 minutes coupled. IES cost is num_reals × (noptmax + 1) runs:

| Configuration | Runs | At 12 workers, 12 min/run | Verdict |
| --- | --- | --- | --- |
| 150 reals × 3 iterations, full period | 600 | ≈ 10 hours | feasible on a server, overnight |
| 150 reals × 3 iterations, full period, 30 % failures | 600 | ≈ 10 h + retries | the realistic case |
| 50 reals × 3 iterations (pilot) | 200 | ≈ 3.5 hours | start here — proves the interface |
| 150 × 3 on a shortened window (e.g. the last 10 years) | 600 | ≈ 2.5 hours | the fallback if the full period is too slow; CdL shortened its window for exactly this reason |
| Aggregated stress periods instead of daily | 600 | much faster | check first whether the soil-moisture group survives aggregation — it probably does not |

Measure before you commit: step 7.4's first job is to time one forward run end to end on the target machine, with the post-processing included. Every number above is an estimate; the calibration window and the ensemble size follow from the measurement, not from the plan.

# 9  Risk register

| Risk | Impact | Mitigation |
| --- | --- | --- |
| UZF PET is not exposed as a writable API array in this MF6 build | WP2 blocked | probe first (--probe equivalent) before coding. Fallback: rewrite the UZF period data per stress period and reload — slower but certain. Trap 5 applies: validate by size, not by the variable list |
| The residual-PET chain double-counts or starves an ET source | wrong ET, silently | the PET-closure test (2.8), the GWET guard (2.6) and the runtime PET-balance line (2.5b) |
| ETg's residual is computed from the PET written to UZF instead of ETuzf ACTUAL | ETg is starved exactly when the deep unsaturated zone is dry — i.e. when phreatic uptake matters most. Totals look plausible; the sourcing is wrong | step 5 consumes etuzf_actual_prev read back from the UZF budget (2.4, 2.5); the runtime PET-balance line and the closure test both catch a regression |
| CRR's topographic ordering breaks the cell-list contract | coupler and DISV mapping desynchronise | store a permutation at build time, never re-sort; assert it is a permutation; the existing cell-order tests must stay green |
| LAK does not converge coupled (it never has here) | WP4 stalls | carry the CdL settings that fixed it — surfdep smoothing, maxiter 200, stagechg 1e-4, equilibrium initial stage. Bisect: LAK alone before LAK + SFR + MVR |
| The PEST forward run is too slow for a useful ensemble | WP7 unusable | §8.4 — measure, then choose the window and the ensemble size. Pilot at 50 realisations first |
| IES overfits to a checkerboard K field | a data-fitting parameter set, not a physical one | the full CdL anti-checkerboard package from the first build (WP7.5 callout) |
| ATS makes the observation files non-deterministic | instruction files misalign, realisations fail | collapse to period-end rows identically in the reference and in every forward run |
| Geospatial dependencies creep onto the model path | the build stops being portable | the WP1 converter and the repo-hygiene test |
| The Streamlit app becomes a hidden dependency of the model | the model stops running headless from Spyder or from a PEST worker | code/ may never import code/app/ or streamlit — asserted by the hygiene test (WP1b.10), not by intent. Streamlit lives in a separate optional environment group |
| A model run is launched in-line from the app | the app blocks for 11 minutes and the run dies on a browser refresh | launch detached, poll a log file and a status.json (WP1b.6); Streamlit reruns the whole script on every interaction, so nothing long-running may sit in the page body |
| A stale cache silently overrides the configuration | a wasted multi-hour run | configuration hashes on every cache (WP0.6). This has already happened once, on CdL's grid design |
| Voronoi invalidates La Mata's validated baseline | the RMSE, the saved hi_spinup_* state and the MODFLOW-NWT comparison no longer apply like-for-like | accepted deliberately: the structured producer is kept permanently so the regression stays runnable on the grid it was established on. Regenerate the spin-up state on the new mesh (WP1c) |
| Centre-sampling a raster onto larger Voronoi cells throws data away | silently wrong soil and aquifer parameter fields | WP1c.3: area-weighted for continuous fields, majority-vote for zone rasters; verify resampled means against the raster means |
| The figure suite is rewritten for vertex grids | weeks of validated work redone for no scientific gain | WP1c.7: rasterise for display instead, and leave MARMITESplot_v3.py untouched. True polygons live in the Streamlit app |
| WP4 and WP5 each perform their own surgery on the cell list | the Phase-2 cell-list contract breaks and the coupler desynchronises | one shared per-cell surface descriptor built at build time (WP4.6), consumed by both |
| MM-MF6 → master merge conflicts | merge stalls | 7 known modify/delete conflicts on trunk/pyEARTH1D/* (as they are on master); decide pyEARTH1D's fate before merging (WP8.5) |

# 10  Milestones and acceptance checklist

| WP | Milestone | Green when | Est. |
| --- | --- | --- | --- |
| WP0 | Configuration file drives the model | the canonical run reproduces byte-identical MF6 inputs from code/configs/lamata.toml; no flag remains except --config, --set, --run-tag | DONE |
| WP1 | Data tiers separated | a fresh clone + DATA_ROOT runs; the repo-hygiene test passes; the dataset README lists every Tier A file | DONE |
| WP1b | Streamlit interface (stage 1) | one setting serves the app on this machine and on the server; every §6 input is inspectable, shapefiles on a map; a run launches, survives a browser refresh and is reviewed from the browser; the hygiene test forbids code/ importing streamlit | DONE |
| WP1c | One grid path, Voronoi by default | a full coupled La Mata run completes on a Voronoi mesh, mass-conserving, whole figure suite drawn with MARMITESplot_v3.py untouched; kind="structured" still reproduces the pre-WP1c run exactly; the three-rung validation ladder passes | DONE (see note) |
| WP2 | Three-source ET | one-year run splits ET five ways, closure test green, UZF groundwater-ET term exactly zero | 3–4 d |
| WP3 | SFR coupled and validated | one-year and full-period runs converge; outlet hydrograph against observation; mass balance < 0.1 % | 3–4 d |
| WP4 | LAK coupled and validated | stages coherent, LAK budget closes, MM↔LAK exchange live, LAK configuration map drawn | 4–5 d |
| WP5 | CRR live | synthetic mass-balance test exact; La Mata run converges; the change to the outlet hydrograph is quantified and explained | 4–5 d |
| WP6 | Figures and observation exports | --postproc-only --preproc emits everything with no skips; the four observation csvs have deterministic row counts | 3–4 d |
| WP7 | PEST-IES calibrated | prior MC completes with < 30 % failures; IES reduces phi; the parameter field is smooth (Moran's I ≥ 0, < 30 % of pilot points at bounds) | 1–2 wk |
| WP8 | Tests, docs, merge | suite green including the six mf6-binary tests; handoff document rewritten; merge rehearsed | 2–3 d |
| WP9 | CdL as the second case study | MMsurf runs from a daily CdL forcing record; a coupled CdL run converges with SFR/LAK/CRR on; nothing case-specific was added to code/ | gated by data |

The estimates are working days of implementation, excluding the multi-hour and overnight runs, which are Alain's to launch in Spyder.

One qualification on WP1c. WP1c's milestone says "the three-rung validation ladder passes". Rung (a) passes exactly (DIS vs DISV-from-DIS: identical to 1e-9 mm/y on all fourteen terms). Rungs (b) and (c) are INCONCLUSIVE rather than passed, and deliberately so: they cannot be judged from a cold start, where runoff and exfiltration reach 30x precipitation because the water table begins above ground. They need a spin-up state generated on the mesh, then a run of at least one hydrological year on each grid — which is the modeller's to launch. The tool refuses to print PASS or FAIL on a spinning-up pair. See docs/MM-MF6/WP1c8_grid_validation.md.

## Settled on 2026-09-09

Study area — develop on La Mata, deliver on CdL (§1.4, WP9).

Grid — Voronoi by default, one path for both case studies; structured kept permanently as the regression anchor (D5, WP1c).

SFR — the CdL / SFRmaker method from the stream shapefile, with the parameter-source pattern and drainage-scaled width kept as the default width producer (§3a).

Open water — LAK and SFR cells bypass the MM soil column (§4a). On the Voronoi mesh this is wholesale and exact, because each pond gets its own pond-scale cell; only stream cells keep a share.

Drain datums — the outlet drains sit at the bottom of the layer, as in CdL, and only the one cell that becomes the stream outlet is replaced by a reach. DRN-SEEP is excluded and stays at the land surface (§3.3b, trap 15).

CRR — cascade over the land surface, reinfiltration into the topmost soil layer; distinct from the bottom-up rejected-infiltration return, which is already coded (D4).

PET depletion — ETg's residual is computed from ETuzf actual, read back from the UZF budget, never from the demand written to UZF (§2b).

## Open questions still to settle

Q1 — Soil-moisture probe depths — which MM soil layer does each inputObsSM_* column correspond to? The sm observation group cannot be built without this mapping. Needed before WP7.3.

Q2 — Observed ET — is the eddy-covariance record usable directly, or do we take the CdL route (MODIS MOD16A2GF, aggregated to land-cover zones)? This decides §6.3 and the et group. 2026-09-25: there is no eddy-covariance record in the dataset.

Q3 — Calibration window — full 1949 stress periods, or a shortened window as CdL adopted? Answer after the WP7.4 timing measurement, not before it.

Q4 — CdL daily meteorology — the WP9 blocker. A station record for the catchment, a national-network series disaggregated, or a downscaled reanalysis? Worth opening the enquiry now: it has a long lead time and it gates the whole of WP9.

Q5 — grid.voronoi.stream_refine (new, WP1c) — the default here is true, but CdL settled on false (the SFRmaker approach: map the network onto the background) after judging a refined mesh "too refined near streams". Every La Mata run so far used false. It changes ncpl and therefore every downstream number, so decide before rung (c).

Q6 — zone-raster resampling (new, WP1c.3) — majority vote is better per cell but biased in aggregate: on La Mata it misassigns 7.64 % of the active area against centre sampling's 3.66 %, and soil zone 1 drops from 13.1 % of the catchment to 9.5 %. [grid] resample = "centre" is the alternative. Which matters more here, per-cell fidelity or zone proportions?

Q7 — seep.cond on a mesh (new, WP1c) — it is a per-cell conductance and therefore grid-dependent. 10 000 m²/d holds the seepage within ~0.2 m on 50 m cells; the first mesh run wanted ~1.5 × 10⁶. Retune against an equilibrated mesh state, not blind.

# 11  Implementation status — what is done, and how to check it

Written 2026-09-10. WP0, WP1, WP1b (stage 1) and WP1c are implemented and committed on branch MM-MF6_SFR_LAK_CRR. This chapter exists so the work can be VERIFIED rather than taken on trust: every claim below has a command beside it that reproduces it. Where the implementation departed from the plan in the chapters that follow, §11.4 says so and why.

## 11.1  At a glance

| Package | Status | What proves it |
| --- | --- | --- |
| WP0 configuration file | DONE | The ~40 command-line flags are replaced by code/configs/lamata.toml. A structured build is byte-identical to the pre-WP0 one across all 13 MF6 input files (bar flopy's timestamp line). |
| WP1 data tiers + converter | DONE | code/tools/gis_to_dataset.py turns the GIS layers into four GRID-INDEPENDENT tables in example/LaMata/. No shapefile is read by the model. test_repo_hygiene.py asserts it. |
| WP1b Streamlit interface | DONE (stage 1) | Five pages, driven in a real browser. A run can be launched from the UI and survives closing the tab. Stages 2–3 (richer Results, Calibration) follow WP6/WP7. |
| WP1c one grid path | DONE (1c.1 – 1c.8) | A coupled La Mata run completes on a Voronoi mesh, mass-conserving (cumulative discrepancy 0.000 % with SFR). 60 figures draw, against 61 on the structured grid. MARMITESplot_v3.py was not touched. |
| WP2 … WP9 | NOT STARTED | Unchanged from the plan below. |

The one thing that is NOT done: Voronoi is not the default. [grid] kind is still "structured", and it stays that way until rung (c) of the validation ladder is judged on an equilibrated multi-year run — which is yours to launch. See §11.3 step 6 and docs/MM-MF6/WP1c8_grid_validation.md.

## 11.2  Before anything: activate the environment

This matters more than it looks. Calling envs\flopy\python.exe directly does NOT put the environment's Library\bin on PATH, so every delay-loaded native DLL fails to resolve and Windows kills the interpreter with 0xC06D007F and no Python traceback. It hits numpy's MKL BLAS — np.dot alone is enough — and therefore matplotlib and rasterio too. Two days of "pre-existing environment faults" were this.

# PowerShell — always by PREFIX (there are TWO envs called flopy; the other is empty)
& "C:\miniconda3\Scripts\conda.exe" run -p C:\miniconda3\envs\flopy --no-capture-output python -u <script>

# or activate the shell once, then plain python works
conda activate C:\miniconda3\envs\flopy

## 11.3  Six checks, in order

1.  The test suite. Expect 353 passed / 6 skipped. If you see ~218 with two modules excluded, the environment was not activated (see §11.2).

cd E:\00code\MM-MF6_SFR_LAK_CRR
python -m pytest code/tests -q

2.  WP0 — the structured grid still writes the same model. This is the regression anchor: everything WP1c added must leave it untouched. Build into a scratch workspace and compare against a known-good one; all 13 files match once flopy's timestamp comment is ignored.

python code/tests/run_lamata_mf6.py --config code/configs/lamata.toml \
       --set run.build_only=true --set run.nsp=10 --set paths.ws=<TMP>

3.  WP1 — the dataset is grid-independent. inputSTREAM.csv, inputSTREAM_param.csv, inputPONDS.csv and inputWATERSHED.csv carry model-CRS geometry with a provenance header naming the shapefile and its mtime. Re-generate them and confirm nothing changes:

python code/tools/gis_to_dataset.py --case LaMata --dry-run

4.  WP1c — a coupled run on a Voronoi mesh. Takes a couple of minutes at 10 stress periods. Watch for the mesh line (ncpl, achieved cell size, resampling and coverage), then the solution check.

python code/tests/run_lamata_mf6.py --config code/configs/lamata.toml \
       --set grid.kind=voronoi --set grid.voronoi.stream_refine=false \
       --set run.nsp=10 --set paths.libmf6=auto \
       --set spinup.strt_heads= --set spinup.steady_means= \
       --set postproc.enable=true --set postproc.preproc=true

mesh: voronoi, ncpl=989 — cell area mean 9845 m², i.e. 99.2 m against the 100 m asked for.

resample=auto, 9.6 source cell(s) per mesh cell, mesh tiles 99.863 % of the grid rectangle.

solution check: converged, cumulative discrepancy 0.0200%.

native MARMITESplot: 60 figure(s) in <ws-root>/out_<stamp>_<tag>/_output/.

5.  WP1c.8 — rung (a) of the validation ladder. DIS against DISV-from-DIS: the same geometry re-expressed, so it must be identical. It is — all fourteen water-balance terms agree to 1e-9 mm/y.

python code/tools/validate_grid.py --ladder --rungs a --nsp 10

6.  The remaining judgement — rungs (b) and (c). These cannot be settled from a cold start: at 10 stress periods runoff and exfiltration reach 30× precipitation, because the water table begins above ground and recirculates. The tool detects that and reports INCONCLUSIVE rather than a misleading PASS or FAIL. To settle them you need a spin-up state generated ON THE MESH (the saved hi_spinup_*.asc is structured, and WP0.6 refuses it on a mesh — correctly), then a run of at least one hydrological year on each grid.

python code/tools/validate_grid.py --compare <WS_structured> <WS_voronoi> \
       --labels structured voronoi --tol 15 --report rung_c.md

## 11.4  The front end — how to launch it

Streamlit is installed in the flopy environment (checked first: the conda solve touched nothing in the model stack). There is no second environment to manage.

cd E:\00code\MM-MF6_SFR_LAK_CRR
streamlit run code/app/Home.py

# it prints a local URL, normally http://localhost:8501 , and opens a browser
# to serve it from the server instead, run it THERE and reach it over the LAN:
streamlit run code/app/Home.py --server.address 0.0.0.0 --server.port 8501

| Page | What it is for |
| --- | --- |
| Home | Case selector, machine paths (green = found), recent runs. |
| Inputs | Tier A — every file the model reads, with size and a viewer (rasters as images, CSVs as tables with their provenance header). Tier B — the source cartography on an OpenStreetMap backdrop, with the CRS of each layer REPORTED rather than guessed. Also runs the WP1 converter, preview first. |
| Configuration | The TOML schema as widgets, every field with its units and meaning — including the 100 MMsurf parameters enumerated from __inputMMsurf.ini, shown pre-filled. |
| Run | Launch a run with one-off section.key=value overrides, validated BEFORE launching. The run is detached: it survives closing the browser. Live log and status. |
| Results | The figures a run wrote, with folder and name filters. |
| Grid | New (WP1c.7). The TRUE cell polygons of the mesh — not the display raster the figure suite uses — coloured by cell area, with the stream network, ponds, catchment boundary and observation points overlaid, plus the shared-face topology report. Pick voronoi in the *Grid kind* selector. |

If the Grid page says "no mesh cached": the mesh is built the first time a run uses that grid, and cached under <ws-root>/MF6_ws_<kind>/_mesh/. Run check 4 of §11.3 once and it will appear. A structured grid has no mesh to show, which is why the page opens empty on the default configuration.

## 11.5  Where the implementation departed from the plan

| Plan said | What was done | Why |
| --- | --- | --- |
| WP1c.1 proves the mesh path with a quadtree, then WP1c.2 adds Voronoi. | Voronoi first; quadtree works too but landed second. | flopy's GRIDGEN wrapper needs pyshp to read its own output back, and the environment is not writable without elevation. Voronoi needs only triangle.exe, which was already present. pyshp has since been installed and the quadtree producer works (ncpl 7728, refined to 12.5 m along the streams). |
| Cells (cid, cid, 0, icell2d) over (ncell, 1) column vectors (the Phase-4 refined_cell_list). | nrow := ncpl, ncol := 1 — arrays are (ncpl, 1), and the ROW INDEX IS the icell2d. | Because i*ncol+j == i, build_cell_list, clsMF6._cellid and clsMF6._griddata all work with NO change. The compacted convention needs an explicit node map and breaks the _griddata reshape. |
| code/ppMF6/marmites_voronoi.py | code/marmites_meshes.py (the producers) + code/marmites_mesh.py (the projection). | The producers are not Voronoi-specific — one function per [grid] kind, all returning the same MF6 DISV gridprops — and the projection is a separate concern from producing a mesh. |
| WP1c.7 rasterises the result vectors for plotLAYER. | That, plus three more figure families that failed for unrelated reasons. | The Sankey was reading <name>.dis.grb, which a DISV model never writes; the obs time series passed the wrong cellid arity to flopy; and delc × delr gives 1 m² per cell on a mesh. None of them was the display raster. |
| The MODFLOW-NWT comparison runs on every run. | It is SKIPPED on a mesh, with the reason printed. | The NWT reference is a 65 × 60 structured run. Comparing it to a mesh is not like-for-like, exactly as §WP1c already anticipated; the new-run figures still draw. |

## 11.6  Decisions waiting on you

| # | Question | Where it bites |
| --- | --- | --- |
| 1 | grid.voronoi.stream_refine defaults to true (this cookbook's choice), but CdL settled on false — the SFRmaker approach, mapping the network onto the background — after judging a refined mesh "too refined near streams". Every La Mata run so far used false. | WP1c / WP3. Changing it changes ncpl and therefore every downstream number. |
| 2 | Zone rasters: majority vote is better PER CELL but biased in aggregate. On La Mata it misassigns 7.64 % of the active area against centre sampling's 3.66 %, and soil zone 1 drops from 13.1 % of the catchment to 9.5 %. [grid] resample = "centre" is the alternative. | WP1c.3. Affects ETsoil and percolation through the soil parameter set. |
| 3 | seep.cond is per cell and therefore grid-dependent. 10 000 m²/d holds the seepage within ~0.2 m on 50 m cells; the first mesh run wanted ~1.5 × 10⁶. | WP1c.8 rungs (b) and (c). Retune it against an equilibrated mesh state, not blind. |
| 4 | Two observation points, SM and EC, fall in the same cell on the 100 m Voronoi mesh, so their modelled series are identical by construction. | WP7 calibration: two observations constraining one cell. |

# Appendix A  Flag → configuration key map

| Current flag | Configuration key | Current flag | Configuration key |
| --- | --- | --- | --- |
| --ws-root | mm_paths.WS_ROOT | --strt-heads | spinup.strt_heads |
| --ws | paths.ws | --steady-means | spinup.steady_means |
| --run-tag | meta.name | --save-strt | spinup.save_strt |
| --libmf6 | mm_paths.LIBMF6 | --save-means | spinup.save_means |
| --nlay | layers.nlay | --spinup | spinup.cycles |
| --aggregate | layers.aggregate | --spinup-tol | spinup.tol |
| --grid | grid.kind (new values) | --strt-dem | spinup.strt_dem |
| --mode | run.mode | --postproc | postproc.enable |
| --relax | run.relax | --preproc | postproc.preproc |
| --nsp | run.nsp | --postproc-only | postproc.only |
| --aggregated | run.daily = false | --gis-ws | paths.gis_ws |
| --no-ats | run.ats = false | --sankey-min-flux | postproc.sankey_min_flux |
| --seep | seep.kind | --no-sankey-full | postproc.sankey_full = false |
| --seep-cond | seep.cond | --map-days | postproc.map_days |
| --uzf-vks-scale | uzf.vks_scale | --sankey-obs-years | postproc.sankey_obs_years |
| --sfr | sfr.enable | --build-only | run.build_only |
| --sfr-rhk | sfr.rhk | --standalone | run.standalone |
| --lak | lak.enable + lak.source | --probe | (stays a CLI diagnostic) |
| --lak-bedleak | lak.bedleak | --max-discrepancy | run.max_discrepancy |
| --daily | run.daily | --allow-bad-budget | run.allow_bad_budget |

New keys with no flag today: the whole of [et], [crr], [ui], [pest] and [meta]; the [grid.voronoi] block and grid.rebuild (WP1c); the [sfr.*] parameter-source sub-tables — width (with its drainage producer), manning, rhk, rbth — plus sfr.min_slope, sfr.source, sfr.monotonic_bed, lak.surfdep, lak.maxiter, lak.stagechg, lak.source. Note grid.kind now takes voronoi | structured | quadtree rather than the old dis | disv.

# Appendix B  Front-end → TOML → model code

One row per key of the run configuration: the panel that asks it, whether a widget actually draws it, and the model code that reads it. It answers one question -- does changing this on the panel change what a run does? -- because a panel field the run ignores is worse than no field: the modeller believes they have set something.

History. The first audit (2026-09-23) found 29 keys asked on a panel and read by nothing, among them the whole Soil panel and the whole State variables panel. This revision (2026-09-24) follows the four fixes that came of it.

Soil panel: the soil column is two tables in the TOML, read by MMsoil; the soil zones, thickness and vegetation cover come from the panel's raster, polygon layer or value, through a new exact area overlay.

Coupling: the coupling mode, under-relaxation, adaptive time stepping and the mass-balance limits are asked on the Run panel.

State variables panel: the point table, the series prefixes and the point name column are the ones the post-processing reads.

Plots panel: the hydrological year, the tick density and the water-balance unit reach the figures, and the four figure switches gate them.

How it was established. Every page rendered headlessly and its widget keys collected (DRAWN); the model code searched, comments excluded, for literal reads, reads through an alias, getattr and whole-block consumption (READ); every key the search could not place settled by reading the code. The list of keys NOT read is code/tests/test_wiring.py, which fails if a new key is read by nothing or a listed key becomes read -- so this table and the guard cannot drift apart.

## Summary

| Status | Keys | Meaning |
| --- | --- | --- |
| WIRED | 136 | a run reads it: changing it on the panel changes the run |
| WIRED (converter) | 6 | read by tools/gis_to_dataset.py, which writes the dataset files the run reads |
| BY DESIGN | 7 | not a model input -- a file-format version, provenance, or a front-end setting |
| AHEAD OF MODEL | 6 | asked for a work package the run does not implement yet |
| NOT WIRED | 12 | the run ignores it (code/tests/test_wiring.py, NOT_WIRED) |
| Total | 167 |  |

## What is still not wired

12 keys, none of them on the soil, groundwater-level or plotting path any more. Most belong to packages not yet exercised on La Mata -- SFR and LAK are off (WP3, WP4) -- and are best wired with them:

| Panel | Key | Why it is not read |
| --- | --- | --- |
| 2 Surface and driving forces | surface.meteo_zones | no reader in marmites_surface |
| 4 Unsaturated zone and groundwater | et.extwc_source | the build always takes extwc from thtr |
| 4 Unsaturated zone and groundwater | lak.maxiter | no reader |
| 4 Unsaturated zone and groundwater | lak.polygons | no reader |
| 4 Unsaturated zone and groundwater | lak.source | the run reads lak.geometry instead |
| 4 Unsaturated zone and groundwater | lak.stagechg | no reader |
| 4 Unsaturated zone and groundwater | lak.surfdep | no reader |
| 4 Unsaturated zone and groundwater | sfr.min_slope | no reader |
| 4 Unsaturated zone and groundwater | sfr.monotonic_bed | no reader |
| 4 Unsaturated zone and groundwater | sfr.source | the network comes from the dataset CSV |
| 5 State variables (calibration) | obs.aet_prefix | no reader takes an actual-ET series yet |
| — | paths.nwt_reference | defined only |

Read by the run but on NO panel -- set in the TOML only: meta.name, paths.case, paths.ws, paths.gis_ws, run.standalone, run.probe, spinup.strt_dem.

## The full table

| Panel | TOML key | On a panel | Status | Read by |
| --- | --- | --- | --- | --- |
| 0 Overview | meta.model | yes | WIRED | tests/run_lamata_mf6.py -- cfg.meta.model_name() |
| 1 Grid | grid.boundary | yes | WIRED | marmites_meshes.py; ppMF6/marmites_props.py |
| 1 Grid | grid.buffer | yes | WIRED | marmites_meshes.py; tools/gis_to_dataset.py |
| 1 Grid | grid.cell_size | yes | WIRED | marmites_meshes.py |
| 1 Grid | grid.crs_epsg | yes | WIRED | marmites_surface.py |
| 1 Grid | grid.dem | yes | WIRED | tests/run_lamata_mf6.py; tools/gis_to_dataset.py |
| 1 Grid | grid.kind | yes | WIRED | marmites_meshes.py; tools/validate_grid.py |
| 1 Grid | grid.override.enable | yes | WIRED | marmites_meshes.py |
| 1 Grid | grid.override.ncol | yes | WIRED | marmites_meshes.py |
| 1 Grid | grid.override.nrow | yes | WIRED | marmites_meshes.py |
| 1 Grid | grid.override.xllcorner | yes | WIRED | marmites_meshes.py |
| 1 Grid | grid.override.yllcorner | yes | WIRED | marmites_meshes.py |
| 1 Grid | grid.ponds | yes | WIRED (converter) | tools/gis_to_dataset.py |
| 1 Grid | grid.quadtree.pond_level | yes | WIRED | marmites_meshes.py (whole block) |
| 1 Grid | grid.quadtree.refine_level | yes | WIRED | marmites_meshes.py (whole block) |
| 1 Grid | grid.quadtree.refine_ponds | yes | WIRED | marmites_meshes.py |
| 1 Grid | grid.quadtree.refine_streams | yes | WIRED | marmites_meshes.py |
| 1 Grid | grid.resample | yes | WIRED | marmites_mesh.py; tests/run_lamata_mf6.py |
| 1 Grid | grid.streams | yes | WIRED (converter) | tools/gis_to_dataset.py |
| 1 Grid | grid.voronoi.cell_far | yes | WIRED | marmites_meshes.py |
| 1 Grid | grid.voronoi.cell_near_stream | yes | WIRED | marmites_meshes.py |
| 1 Grid | grid.voronoi.cell_pond | yes | WIRED | marmites_meshes.py |
| 1 Grid | grid.voronoi.grade_ratio | yes | WIRED | marmites_meshes.py (whole block) |
| 1 Grid | grid.voronoi.refine_ponds | yes | WIRED | marmites_meshes.py |
| 1 Grid | grid.voronoi.stream_buffer | yes | WIRED | marmites_meshes.py |
| 1 Grid | grid.voronoi.stream_refine | yes | WIRED | marmites_meshes.py; tools/validate_grid.py |
| 1 Grid | grid.voronoi.trans_levels | yes | WIRED | marmites_meshes.py (whole block) |
| 2 Surface and driving forces | surface.crop | yes | WIRED | marmites_surface.py |
| 2 Surface and driving forces | surface.crop_schedule | yes | WIRED | marmites_surface.py |
| 2 Surface and driving forces | surface.irr_ts | yes | WIRED | marmites_surface.py |
| 2 Surface and driving forces | surface.irr_zones | yes | WIRED (converter) | tools/gis_to_dataset.py |
| 2 Surface and driving forces | surface.irrigation | yes | WIRED | marmites_surface.py |
| 2 Surface and driving forces | surface.meteo_ts | yes | WIRED | marmites_surface.py |
| 2 Surface and driving forces | surface.meteo_zones | yes | NOT WIRED | no reader in marmites_surface |
| 2 Surface and driving forces | surface.nfield | yes | WIRED | marmites_surface.py |
| 2 Surface and driving forces | surface.out_prefix | yes | WIRED | marmites_surface.py |
| 2 Surface and driving forces | surface.plot | yes | WIRED | marmites_surface.py |
| 2 Surface and driving forces | surface.soil | yes | WIRED | marmites_surface.py |
| 2 Surface and driving forces | surface.station | yes | WIRED | marmites_surface.py |
| 2 Surface and driving forces | surface.vegetation | yes | WIRED | marmites_surface.py |
| 3 Soil | soil.horizon | yes | WIRED | ppMF6/marmites_props.py |
| 3 Soil | soil.thickness | yes | WIRED | tests/run_lamata_mf6.py |
| 3 Soil | soil.veg_class | yes | WIRED | ppMF6/marmites_props.py |
| 3 Soil | soil.veg_column | yes | WIRED | ppMF6/marmites_props.py |
| 3 Soil | soil.veg_layer | yes | WIRED | ppMF6/marmites_props.py; tools/gis_to_dataset.py |
| 3 Soil | soil.zone | yes | WIRED | ppMF6/marmites_props.py |
| 3 Soil | soil.zones | yes | WIRED | ppMF6/marmites_props.py; tests/run_lamata_mf6.py |
| 4 Unsaturated zone and groundwater | crr.beta | yes | AHEAD OF MODEL | WP5 -- CRR is not built in the MARMITES run yet |
| 4 Unsaturated zone and groundwater | crr.dem | yes | AHEAD OF MODEL | WP5 -- CRR is not built in the MARMITES run yet |
| 4 Unsaturated zone and groundwater | crr.enable | yes | AHEAD OF MODEL | WP5 -- CRR is not built in the MARMITES run yet |
| 4 Unsaturated zone and groundwater | crr.sinks | yes | AHEAD OF MODEL | WP5 -- CRR is not built in the MARMITES run yet |
| 4 Unsaturated zone and groundwater | drn.at_layer_base | yes | WIRED | ppMF6/marmites_props.py; tests/run_lamata_mf6.py |
| 4 Unsaturated zone and groundwater | drn.cond | yes | WIRED | ppMF6/marmites_props.py |
| 4 Unsaturated zone and groundwater | drn.elevation | yes | WIRED | ppMF6/marmites_props.py |
| 4 Unsaturated zone and groundwater | drn.enable | yes | WIRED | ppMF6/marmites_props.py -- apply_boundaries, pkg.enable |
| 4 Unsaturated zone and groundwater | drn.layers | yes | WIRED | ppMF6/marmites_props.py -- apply_boundaries, pkg.layers |
| 4 Unsaturated zone and groundwater | et.extdp | yes | WIRED | tests/run_lamata_mf6.py |
| 4 Unsaturated zone and groundwater | et.extwc_source | yes | NOT WIRED | the build always takes extwc from thtr |
| 4 Unsaturated zone and groundwater | et.unsat_form | yes | WIRED | tests/run_lamata_mf6.py |
| 4 Unsaturated zone and groundwater | ghb.cond | yes | WIRED | ppMF6/marmites_props.py |
| 4 Unsaturated zone and groundwater | ghb.enable | yes | WIRED | ppMF6/marmites_props.py -- apply_boundaries, pkg.enable |
| 4 Unsaturated zone and groundwater | ghb.head | yes | WIRED | ppMF6/marmites_props.py |
| 4 Unsaturated zone and groundwater | ghb.layers | yes | WIRED | ppMF6/marmites_props.py -- apply_boundaries, pkg.layers |
| 4 Unsaturated zone and groundwater | lak.bedleak | yes | WIRED | tests/run_lamata_mf6.py |
| 4 Unsaturated zone and groundwater | lak.depth | yes | WIRED | tests/run_lamata_mf6.py |
| 4 Unsaturated zone and groundwater | lak.enable | yes | WIRED | tests/run_lamata_mf6.py; tools/flags_to_toml.py |
| 4 Unsaturated zone and groundwater | lak.geometry | yes | WIRED | tests/run_lamata_mf6.py |
| 4 Unsaturated zone and groundwater | lak.maxiter | yes | NOT WIRED | no reader |
| 4 Unsaturated zone and groundwater | lak.polygons | yes | NOT WIRED | no reader |
| 4 Unsaturated zone and groundwater | lak.source | yes | NOT WIRED | the run reads lak.geometry instead |
| 4 Unsaturated zone and groundwater | lak.stagechg | yes | NOT WIRED | no reader |
| 4 Unsaturated zone and groundwater | lak.surfdep | yes | NOT WIRED | no reader |
| 4 Unsaturated zone and groundwater | layers.convertible | yes | WIRED | ppMF6/marmites_props.py -- PROPERTIES / FLAGS tables, getattr(cfg.layers, name) |
| 4 Unsaturated zone and groundwater | layers.hnoflo | yes | WIRED | ppMF6/marmites_props.py; tests/run_lamata_mf6.py |
| 4 Unsaturated zone and groundwater | layers.ibound | yes | WIRED | ppMF6/marmites_props.py; tests/run_lamata_mf6.py |
| 4 Unsaturated zone and groundwater | layers.k | yes | WIRED | ppMF6/marmites_props.py -- PROPERTIES / FLAGS tables, getattr(cfg.layers, name) |
| 4 Unsaturated zone and groundwater | layers.k33 | yes | WIRED | ppMF6/marmites_props.py -- PROPERTIES / FLAGS tables, getattr(cfg.layers, name) |
| 4 Unsaturated zone and groundwater | layers.k33_as_ratio | yes | WIRED | ppMF6/marmites_props.py -- PROPERTIES / FLAGS tables, getattr(cfg.layers, name) |
| 4 Unsaturated zone and groundwater | layers.nlay | yes | WIRED | ppMF6/marmites_props.py; tests/run_lamata_mf6.py |
| 4 Unsaturated zone and groundwater | layers.ss | yes | WIRED | ppMF6/marmites_props.py -- PROPERTIES / FLAGS tables, getattr(cfg.layers, name) |
| 4 Unsaturated zone and groundwater | layers.sy | yes | WIRED | ppMF6/marmites_props.py -- PROPERTIES / FLAGS tables, getattr(cfg.layers, name) |
| 4 Unsaturated zone and groundwater | layers.thickness | yes | WIRED | ppMF6/marmites_props.py -- PROPERTIES / FLAGS tables, getattr(cfg.layers, name) |
| 4 Unsaturated zone and groundwater | seep.cond | yes | WIRED | tests/run_lamata_mf6.py |
| 4 Unsaturated zone and groundwater | seep.kind | yes | WIRED | tests/run_lamata_mf6.py |
| 4 Unsaturated zone and groundwater | sfr.depth | yes | WIRED | ppMF6/marmites_channel.py; tests/run_lamata_mf6.py |
| 4 Unsaturated zone and groundwater | sfr.enable | yes | WIRED | tests/run_lamata_mf6.py |
| 4 Unsaturated zone and groundwater | sfr.manning | yes | WIRED (converter) | tools/gis_to_dataset.py |
| 4 Unsaturated zone and groundwater | sfr.min_slope | yes | NOT WIRED | no reader |
| 4 Unsaturated zone and groundwater | sfr.monotonic_bed | yes | NOT WIRED | no reader |
| 4 Unsaturated zone and groundwater | sfr.rbth | yes | WIRED (converter) | tools/gis_to_dataset.py |
| 4 Unsaturated zone and groundwater | sfr.rhk | yes | WIRED | tests/run_lamata_mf6.py; tools/flags_to_toml.py |
| 4 Unsaturated zone and groundwater | sfr.source | yes | NOT WIRED | the network comes from the dataset CSV |
| 4 Unsaturated zone and groundwater | sfr.width | yes | WIRED | ppMF6/marmites_channel.py; tests/run_lamata_mf6.py |
| 4 Unsaturated zone and groundwater | spinup.cycles | yes | WIRED | tests/run_lamata_mf6.py; tools/validate_grid.py |
| 4 Unsaturated zone and groundwater | spinup.save_means | yes | WIRED | tests/run_lamata_mf6.py |
| 4 Unsaturated zone and groundwater | spinup.save_strt | yes | WIRED | tests/run_lamata_mf6.py |
| 4 Unsaturated zone and groundwater | spinup.steady_means | yes | WIRED | tests/run_lamata_mf6.py; tools/validate_grid.py |
| 4 Unsaturated zone and groundwater | spinup.strt_dem | no | WIRED | ppMF6/marmites_props.py; tests/run_lamata_mf6.py |
| 4 Unsaturated zone and groundwater | spinup.strt_heads | yes | WIRED | ppMF6/marmites_props.py; tests/run_lamata_mf6.py |
| 4 Unsaturated zone and groundwater | spinup.tol | yes | WIRED | tests/run_lamata_mf6.py |
| 4 Unsaturated zone and groundwater | uzf.eps | yes | WIRED | ppMF6/marmites_props.py -- apply_uzf name table -> ppMF6/marmites_mf6.py |
| 4 Unsaturated zone and groundwater | uzf.ntrailwaves | yes | WIRED | ppMF6/marmites_props.py |
| 4 Unsaturated zone and groundwater | uzf.nwavesets | yes | WIRED | ppMF6/marmites_props.py |
| 4 Unsaturated zone and groundwater | uzf.surfdep | yes | WIRED | ppMF6/marmites_props.py -- apply_uzf name table -> ppMF6/marmites_mf6.py |
| 4 Unsaturated zone and groundwater | uzf.thti | yes | WIRED | ppMF6/marmites_props.py -- apply_uzf name table -> ppMF6/marmites_mf6.py |
| 4 Unsaturated zone and groundwater | uzf.thtr | yes | WIRED | ppMF6/marmites_props.py -- apply_uzf name table -> ppMF6/marmites_mf6.py |
| 4 Unsaturated zone and groundwater | uzf.thtr_from | yes | WIRED | ppMF6/marmites_mf6.py; ppMF6/marmites_props.py |
| 4 Unsaturated zone and groundwater | uzf.thts | yes | WIRED | ppMF6/marmites_props.py -- apply_uzf name table -> ppMF6/marmites_mf6.py |
| 4 Unsaturated zone and groundwater | uzf.vks | yes | WIRED | ppMF6/marmites_props.py |
| 4 Unsaturated zone and groundwater | uzf.vks_from | yes | WIRED | ppMF6/marmites_props.py |
| 4 Unsaturated zone and groundwater | uzf.vks_scale | yes | WIRED | tests/run_lamata_mf6.py |
| 5 State variables (calibration) | obs.aet_prefix | yes | NOT WIRED | no reader takes an actual-ET series yet |
| 5 State variables (calibration) | obs.heads_prefix | yes | WIRED | ppMF6/marmites_postprocess.py |
| 5 State variables (calibration) | obs.layer | yes | WIRED (converter) | tools/gis_to_dataset.py |
| 5 State variables (calibration) | obs.name_column | yes | WIRED | ppMF6/marmites_postprocess.py |
| 5 State variables (calibration) | obs.ro_prefix | yes | WIRED | ppMF6/marmites_postprocess.py |
| 5 State variables (calibration) | obs.sm_prefix | yes | WIRED | ppMF6/marmites_postprocess.py |
| 5 State variables (calibration) | obs.table | yes | WIRED | ppMF6/marmites_postprocess.py |
| 6 Plots | postproc.enable | yes | WIRED | tests/run_lamata_mf6.py; tools/flags_to_toml.py |
| 6 Plots | postproc.hydro_year_start | yes | WIRED | ppMF6/marmites_props.py |
| 6 Plots | postproc.input_maps | yes | WIRED | tests/run_lamata_mf6.py |
| 6 Plots | postproc.map_days | yes | WIRED | tests/run_lamata_mf6.py |
| 6 Plots | postproc.obs_series | yes | WIRED | tests/run_lamata_mf6.py |
| 6 Plots | postproc.only | yes | WIRED | tests/run_lamata_mf6.py; tools/flags_to_toml.py |
| 6 Plots | postproc.preproc | yes | WIRED | tests/run_lamata_mf6.py; tools/validate_grid.py |
| 6 Plots | postproc.result_maps | yes | WIRED | tests/run_lamata_mf6.py |
| 6 Plots | postproc.sankey | yes | WIRED | tests/run_lamata_mf6.py |
| 6 Plots | postproc.sankey_full | yes | WIRED | tests/run_lamata_mf6.py |
| 6 Plots | postproc.sankey_min_flux | yes | WIRED | tests/run_lamata_mf6.py |
| 6 Plots | postproc.sankey_obs_years | yes | WIRED | tests/run_lamata_mf6.py |
| 6 Plots | postproc.tick_semester_years | yes | WIRED | ppMF6/marmites_props.py |
| 6 Plots | postproc.tick_trimester_years | yes | WIRED | ppMF6/marmites_props.py |
| 6 Plots | postproc.wb_unit | yes | WIRED | MARMITESutilities/MARMITESplot/MARMITESplot_v3.py; ppMF6/marmites_props.py |
| 8 Run | paths.libmf6 | yes | WIRED | tests/run_lamata_mf6.py; tools/flags_to_toml.py |
| 8 Run | solver.complexity | yes | WIRED | tests/run_lamata_mf6.py |
| 8 Run | solver.inner_dvclose | yes | WIRED | tests/run_lamata_mf6.py |
| 8 Run | solver.inner_rclose | yes | WIRED | tests/run_lamata_mf6.py |
| 8 Run | solver.outer_dvclose | yes | WIRED | tests/run_lamata_mf6.py |
| 8 Run | solver.outer_maximum | yes | WIRED | tests/run_lamata_mf6.py |
| — | meta.config_version | no | BY DESIGN | file-format version, checked by validate() |
| — | meta.description | no | BY DESIGN | provenance: copied into resolved_config.toml |
| — | meta.name | no | WIRED | tests/run_lamata_mf6.py |
| — | paths.case | no | WIRED | tests/run_lamata_mf6.py |
| — | paths.gis_ws | no | WIRED | tests/run_lamata_mf6.py; tools/flags_to_toml.py |
| — | paths.nwt_reference | no | NOT WIRED | defined only |
| — | paths.ws | no | WIRED | tests/run_lamata_mf6.py; tools/flags_to_toml.py |
| — | pest.params_npz | no | AHEAD OF MODEL | WP7 -- PEST++-IES is not built yet |
| — | pest.use_pest_params | no | AHEAD OF MODEL | WP7 -- PEST++-IES is not built yet |
| — | run.allow_bad_budget | yes | WIRED | tests/run_lamata_mf6.py |
| — | run.ats | yes | WIRED | tests/run_lamata_mf6.py |
| — | run.build_only | yes | WIRED | tests/run_lamata_mf6.py; tools/validate_grid.py |
| — | run.daily | yes | WIRED | tests/run_lamata_mf6.py; tools/flags_to_toml.py |
| — | run.max_discrepancy | yes | WIRED | tests/run_lamata_mf6.py |
| — | run.mode | yes | WIRED | tests/run_lamata_mf6.py |
| — | run.model | yes | WIRED | tests/run_lamata_mf6.py |
| — | run.nsp | yes | WIRED | tests/run_lamata_mf6.py; tools/validate_grid.py |
| — | run.perlen_max | yes | WIRED | tests/run_lamata_mf6.py |
| — | run.plot | yes | WIRED | tests/run_lamata_mf6.py |
| — | run.probe | no | WIRED | tests/run_lamata_mf6.py |
| — | run.relax | yes | WIRED | tests/run_lamata_mf6.py |
| — | run.standalone | no | WIRED | tests/run_lamata_mf6.py |
| — | run.surface | yes | WIRED | marmites_surface.py; tests/run_lamata_mf6.py |
| — | ui.address | no | BY DESIGN | front-end setting (Run panel / launcher) |
| — | ui.execution | no | BY DESIGN | front-end setting (Run panel / launcher) |
| — | ui.poll_secs | no | BY DESIGN | front-end setting (Run panel / launcher) |
| — | ui.port | no | BY DESIGN | front-end setting (Run panel / launcher) |
| — | ui.runs_dir | no | BY DESIGN | front-end setting (Run panel / launcher) |