MARMITES – MODFLOW 6

Input data required to set up a catchment model

Version of 6 October 2026, code branch MM-MF6_SFR_LAK_CRR

# 1. General conventions

- Coordinate system. One projected, metric CRS for every spatial file (give the EPSG code). Shapefiles must carry their .prj.

- Vector data: ESRI shapefile (all components). Rasters: any GDAL-readable format (GeoTIFF preferred, ESRI ASCII accepted).

- Spatially distributed parameters can each be given in one of three ways: a raster, an attribute column of a polygon layer, or a single uniform value.

- Time series: plain text, whitespace-separated, one row per record, date first as YYYY-MM-DD. Missing values: leave the row out, or write a value of 9999 or more (any |value| ≥ 9000 is read as missing).

- Period covered: the meteorological forcing must span the whole simulation, plus at least one year before it for the model spin-up. The observations should overlap the simulation.

- Req. column: R = required; C = required if the related process exists in the catchment (e.g. ponds, irrigation); O = optional, used for calibration or refinement.

# 2. Domain and topography

| Item | Format | Content, units, notes | Req. |
| --- | --- | --- | --- |
| Catchment boundary | Polygon shapefile | Outer limit of the active model domain. One polygon. | R |
| Digital elevation model | Raster | Land-surface elevation (m a.s.l.), sink-filled, covering the catchment. Use the finest resolution available (La Mata: 5 m). Gives the land surface, the flow routing and the pond rims. | R |
| Stream network | Line shapefile | Mapped channels, digitised downstream. Becomes the stream-routing network (SFR) and refines the mesh along it. | C |
| Ponds / small reservoirs | Polygon shapefile | One polygon per water body (footprint at full level). Becomes the lake package (LAK). | C |

# 3. Meteorological forcing

| Item | Format | Content, units, notes | Req. |
| --- | --- | --- | --- |
| Meteorological record | Text table, hourly | Columns in this exact order (the header row is required but not read): Date (YYYY-MM-DD) · Time (hh:mm) · P rainfall (mm) · Ta air temperature (°C) · RH relative humidity (%) · Pa air pressure (kPa) · u wind speed (m/s) · Rs incoming shortwave radiation (MJ m⁻² h⁻¹). Several stations: repeat the six variable columns per station, in the same order. No gaps: fill them before delivery and say how. | R |
| Station metadata | Table, one row per station | Name; X, Y (project CRS); altitude (m a.s.l.); height of the wind and of the humidity sensors (m); time zone of the logger and whether it records standard or summer time. | R |
| Meteo zones | Polygon shapefile | Only to override the default (Thiessen polygons of the stations). | O |

Daily-only records cannot be used as they are: the potential evapotranspiration is computed hourly (FAO-56 Penman–Monteith). Tell us if only daily data exist, so a disaggregation can be agreed.

# 4. Vegetation and land cover

| Item | Format | Content, units, notes | Req. |
| --- | --- | --- | --- |
| Vegetation map | Polygon shapefile | Crown or stand polygons with a class column (e.g. species code). Areas without vegetation are bare soil. The cover share per model cell is computed by exact overlay. | R |
| Parameters per vegetation type | Table, one row per type | Height dry / wet season (m) · canopy storage (mm) · maximum leaf conductance (m/s) · LAI dry / wet season (m²/m²) · shelter factor dry / wet (–) · albedo dry / wet (–) · start day of dry and wet season (Julian day) · length of the dry→wet and wet→dry transitions (d) · maximum rooting depth (m). Literature values are acceptable; give the source. | R |

# 5. Irrigation (only where fields are irrigated)

| Item | Format | Content, units, notes | Req. |
| --- | --- | --- | --- |
| Irrigated fields | Polygon shapefile | One polygon per field, column field_id = 1…N. | C |
| Irrigation series | Text table, hourly | Date · Time · one column per field (mm). Same rows (dates and times) as the meteorological record. | C |
| Crop schedule | Text table, one file per field | One row per season: start date · end date · growing duration (d) · wilting duration (d) · crop index. | C |
| Crop parameters | Table, one row per crop | Maximum height (m) · canopy storage (mm) · maximum leaf conductance (m/s) · maximum LAI · shelter factor · albedo · maximum rooting depth (m). | C |
| Irrigation method | Text | Sprinkler, drip or flood, per field. | C |

# 6. Soil

| Item | Format | Content, units, notes | Req. |
| --- | --- | --- | --- |
| Soil zone map | Polygon shapefile | Polygons with an integer column: zone number 1…N. | R |
| Soil thickness | Raster, polygon column or value | Depth of the soil column above the aquifer (m). This is the base of the soil model and the top of the groundwater model. | R |
| Soil horizons per zone | Table, one row per horizon, top to bottom | Zone · share of the soil thickness (–, the horizons of one zone sum to 1) · saturated moisture θs (m³/m³) · field capacity θfc (m³/m³) · residual / wilting moisture θr (m³/m³) · initial moisture (m³/m³) · saturated hydraulic conductivity Ks (mm/d). Measured values (laboratory, pedotransfer functions) preferred; state the method. | R |
| Surface soil per zone | Table, one row per zone | Porosity and field capacity of the top 1 cm (m³/m³) · albedo dry / wet (–). | R |

# 7. Hydrogeology

| Item | Format | Content, units, notes | Req. |
| --- | --- | --- | --- |
| Number of layers | Value | Hydrogeological units represented as model layers, top to bottom. | R |
| Active extent per layer | Raster or polygon, per layer | Where each layer exists (a unit may pinch out inside the catchment). | R |
| Thickness per layer | Raster, column or value, per layer | Thickness (m). The top of the first layer is land surface minus soil thickness. | R |
| Horizontal hydraulic conductivity | Raster, column or value, per layer | K (m/d). | R |
| Vertical anisotropy | Value or raster, per layer | Kh/Kv ratio (–), or Kv (m/d). | R |
| Specific yield | Raster, column or value, per layer | Sy (–). | R |
| Specific storage | Raster, column or value, per layer | Ss (1/m). | R |
| Unsaturated-zone properties | Value per catchment (or per layer) | Saturated and residual water content (m³/m³) and Brooks–Corey exponent (–) of the material between the soil base and the water table. | O |
| Source of each value | Text | Pumping tests, slug tests, geophysics, literature: needed to set the calibration ranges. | R |

# 8. Surface water

| Item | Format | Content, units, notes | Req. |
| --- | --- | --- | --- |
| Channel geometry | Value, or column of the stream layer | Channel width (m) and depth (m), or width as a function of drainage area if that is what is known. | C |
| Streambed | Value or column | Bed thickness (m) · bed hydraulic conductivity (m/d) · Manning’s n (–). | C |
| Ponds | Value or column of the pond layer | Depth at full level (m) · bed leakance (1/d), or bed material if unknown. | C |

# 9. Boundary conditions

| Item | Format | Content, units, notes | Req. |
| --- | --- | --- | --- |
| Outlet drain | Line shapefile + value | Line drawn across the catchment outlet; the cells it crosses on the boundary drain groundwater out. Conductance per metre of boundary (m²/d per m), or the data to estimate it. | R |
| General-head boundary | Line shapefile + values | Where groundwater enters or leaves across the catchment limit: head (m a.s.l.) and conductance (m²/d per m). | C |
| Pumping wells | Point shapefile + table | Location · screened depth (m) or layer · rate (m³/d) with its dates. Not yet handled by the current code version: supply now if they exist; their support is planned. | C |

# 10. Observations (calibration and validation)

| Item | Format | Content, units, notes | Req. |
| --- | --- | --- | --- |
| Observation points | Text table or point shapefile | Name · X · Y · model layer of the screen. One row per piezometer and soil-moisture probe. | R |
| Groundwater heads | One text file per point | Date · head (m a.s.l.). Give the reference elevation and the screen depth of each well. | R |
| Soil moisture | One text file per point | Date · volumetric moisture (m³/m³), one column per soil horizon, top to bottom. State the probe depths and the sensor calibration. | O |
| Streamflow at the outlet | One text file | Date · daily discharge expressed over the catchment area (mm/d), or in m³/s with the gauge location and rating curve. | O |
| Actual evapotranspiration | One text file per site | Date · actual ET (mm/d). Eddy covariance, lysimeter, sap flow, or remote sensing (state the product). | O |

Please return with each dataset: its source, the measurement method, its known uncertainty, and the processing applied (gap filling, smoothing, unit conversion).